- Height of freezing level during monsoon season over Delhi is near

  - 850 mb

  - 700 mb

  - 500 mb

- In break monsoon conditions the axis of the monsoon through at mean sea level is

  - In its normal position

  - To the south of its normal position

  - To the north of its normal position

- If the surface air temperature is 30 deg C the temperature at 2 km. (assuming dry adiabatic lapse rate prevailing up to 2 kilometers from the surface) is about

  - 18 deg C

  - 10 deg C

  - 42 deg C

- Warm an most air moving over a cold ground gives rise to:

  - Thunder Clouds

  - Fog and Stratus

  - Frontal Clouds

- An isothermal atmosphere is

  - Stable

  - Unstable

  - Neutral

- Fronts are associated with

  - Tropical Cyclone

  - Monsoon depression

  - Extra Tropical Cyclones

- The thermal wind is

  - The wind that below because of thermals

  - The wind which must be added vectoraly to the geostrophic wind at the lower level in order to obtain the geostrophic wind at the upper level

  - The ward wind that blows the hill on the leeward side

- A cold front is defined as:

  - A ridge of high pressure

  - Loading edge of advancing warm air

  - Loading edge of advancing cold air

- Normal winds at 200 mb at Delhi in January are

  - Westerlies

  - Easterlies

  - Southerlies

- Sigmet is issued for

  - Fog

  - Heavy Rain

  - Active thunderstorm Area

- A jet stream wind is

  - 40 Kt or more

  - 60 Kt or more

  - Not more then 100 Knot

- Easterly jet over India is seen in

  - Northest monsoon season

  - Pro – monsoon season

  - Southwest monsoon season

- Radio – Sonde is used to measure

  - Amount of cloud and its speed

  - Wind at various pressure levels

  - The temperature and humidity at various pressure levels.

- Tropopause is lowest over:

  - Polar Areas

  - Mid Latitudes

  - Tropics

- The rate of fall of pressure with height on warm air mass will be:

  - The same as in cold airmass

  - More than in a cold airmass

  - Less then in a cold airmass

- Warm air advection gives rise to

  - Veering of wind with height

  - Backing of wind height

  - No change in wind direction with height

- Severe tropical cyclones in bay of Bengal are most frequent in

  - S/W Monsoon Season

  - Pre – Monsoon Season

  - Post – Monsoon Season

- Wind speed in tropical cyclone

  - Is between 24 and 35 knots

  - In not more than 50 knots

  - Exceeds 33 knots

- In a cyclone in Northern Hemisphere, around the low pressure centre

  - Wind blows in an anti clockwise direction

  - Wind blows in clockwise direction

  - Wind are clam

- If an reads aerodrome elevation at ground level, then the sub – scale must have been set to:

  - QNH

  - QNE

  - QFE

- Density of moist air is

  - Higher than dry air

  - Lower than dry air

  - Equal to dry air

- Lines of constant wind speed drown on weather charts are called:

  - Isobars

  - Isotachs

  - Isogons

- In geostrophic wind the following factors are important

  - Pressure gradient force and Corriolis force

  - Pressure gradient force and Centrifugal force

  - Centrifugal force and Corriolis force

- A TREND type landing forecast is valid for

  - 9 hrs

  - 2 hrs

  - 4 hrs

- Match the following by choosing one from A and one from B							A					Matching A

  - Thunderstorms				Eye

  - Depression				Fine Weather

  - Tropical Cyclone 				Extra tropics

  - Frost 					Bad Weather

  - Satellite 					Winter

  - Stable Atmosphere 			Squall

  - Fronts					Cloud Imagery

  - Anticyclones 				Pro – Monsoon

  - Heat waves 				Strong Winds

  - Sub – tropical westerly jet		Inversions

- The average height of the Tropopause varies from approx

  - 8 km at the poles to 16 km at the equator

  - 12 km at the equator to 9 km at the poles

  - 16 km at the poles to 9 km at the equator

- The rate of fall of pressure with height in a warm air mass

  - Will be less than in a cold air mass

  - Will be more than in a cold air mass

  - The same as in a cold a air mass

- In the Northern Hemisphere, geostrophic wind blows

  - Parallel to the isobars with high pressure to the left

  - Across the isobars from high pressure to low pressure

  - Parallel to the isobars with low pressure to the left

- Under the same condition of pressure and temperature moist air is

  - Lighter than dry air

  - Heavier than dry air

  - Has the same density as dry air

- If the wind above a certain station is reported to be from the North at an altitude of about 3600 ft you would expect the surface wind to be from:

  - North East

  - North

  - North West

- The solar radiation received by the earth is called

  - Insolation

  - Convection

  - Inversion

- Earth radiates heat energy in the

  - Long Waves

  - Short Waves

  - Very Short Waves

- Instrument for recording pressure is called:

  - Anemograph

  - Barograph

  - Hygrograph

- Any lapse rate less than saturated adiabatic lapse rate is described as

  - Absolutely unstable

  - Conditionally stable

  - Absolutely stable

- At a height of 16 km the temperature over the equator is

  - Higher than over the poles

  - Nearly the same over the poles

  - Less than over the poles

- On the lee side of mountains you would normally expect to find

  - Down Drafts

  - Updrafts

  - Eddies and down-drafts

- If you are flying from Bombay to Calcutta holding a constant indicated altitude and experience a drift to the Starboard side, your actual altitude will be

  - Lower than your indicated altitude

  - The same as the indicated altitude

  - Higher than the indicated altitude

- Ice generally forms on aircraft when

  - The aircraft is flying through cirrus clouds.

  - Ice crystals are present in the area of flight

  - Water droplets are present with subfreezing temperature.

- High clouds that take the form of hooks ending in small tufts are termed

  - Cirrocumulus

  - Cirrus

  - Altocumulus

- Lines of constant wind speed on weather charts are called

  - Isotachs

  - Isohects

  - Isobars

- Radio Sonde is used to determine

  - The base of low clouds over a station

  - The wind at various levels

  - Temperature and humidity at various pressure levels.

- When a warm front passess, the surface wind

  - Back and increases

  - Veers and decrease

  - Veers and increase

- The density of air

  - Varies directly as pressure and temperature

  - Varies directly as pressure and inversely as temperature.

  - Varies inversely as pressure and directly as temperature

- In the Troposphere density

  - Increase with height

  - Decrease with height

  - Remains nearly constant.

- Norwesters are

  - Depressions in (he Bay of Bengal which move towards

  - Violent thunderstorms

  - Western disturbances which affect NW India.

- The major constituent of air by weight is

  - Oxygen

  - Nitrogen

  - Carbon dioxide

- Thunderstorm activity is predominant in

  - NE India

  - Central India

  - South Peninsular India

- The altimeter of an aircraft parked at an airfield, indicated a height of 450 ft when the subscale of the instrument was set to 1013 Mbs If the QNH a the airfield was 1000-Mb and a change of pressure of 1 Mb represents a height change of 27 ft, what is the elevation of the airfield

  - 99 ft

  - 549 ft

  - 530 ft

- The average height of the troposphere is

  - Greatest near equator and minimum near poles

  - Greatest near middle latitudes and minimum near equator and poles

  - Greatest near poles and minimum near equator.

- An adiabatic process is one in which heat content of the system.

  - Increases

  - Decreases

  - Does not change

- Surface wind velocity reaches a maximum of most days at a given station

  - In the afternoon

  - In the early morning hours

  - Any time of the day.

- The temperature to which moist air must be cooled at its existing pressure
and humidity-mixing ratio in order that it shall be saturated, is called

  - Wet bulb temperature

  - Dew point temperature

  - Virtual temperature

- In the International Standard Atmosphere

  - Means sea level pressure is 1000 mb and temperature is 15 C.

  - Means sea level pressure is 1013.25 mb and temp is 15 C

  - Rate of fall of temp with height is 6.5 C/Km upto 21 Km.

- An area on a weather map where isobars are relatively more closely packed, the surface winds are likely to be

  - Light and parallel to isobars

  - Strong and parallel to isobars

  - Strong and blowing across the isobars

- The winds which spiral inward in a counterclockwise direction in the Northern Hemisphere are associated with

  - Low pressure area

  - High pressure area

  - Turbulence

- Strong wind sheer conditions  in the lower levels of the atmosphere are normally associated with

  - Thunderstorm

  - Amount of moisture present

  - Fine weather

- Hail is most likely to fall from a cloud

  - Having layers

  - Composed of ice crystals

  - Having strong vertical development

- The atmosphere may be nearly taken to be dry if the difference between the dry bulb temp and dew point temperature is

  - 20 deg C

  - 5 deg C

  - 0 deg C

- Dry air having a temperature of 35 deg C on surface when forced to rise adiabatically by 1 Km would attain a temperature of

  - 29 deg C

  - 25 deg C

  - 45 deg C

  - 41 deg C

- Advection fog forms on nights characterized by

  - Light and most winds; sky clear

  - Strong winds; sky cloudy

  - Slow passage of relatively warm, moist and stable air over a cool surface

- If an observer stands with his back to the wind in the Northern Hemisphere, the low-pressure area will be

  - Ahead of him

  - To his left

  - To his right

- A High-intensity, short - duration, showery precipitation is associated with

  - A stratus cloud

  - An altostratus cloud

  - A cumulonimbus cloud

- A squall is a phenomenon characterizing

  - Heavy rainfall and downdraft in a thunderstorm

  - The   sudden   and   frequent   lightening   discharge   observed   in   a
thunderstorm

  - The sudden rise of surface wind satisfying the specified criterion of
Wind speed and duration associated with a thunderstorm.

- The conditions necessary for formation of a thunderstorm are

  - Steep lapse rate, strong winds

  - Steep lapse rate, adequate supply of moisture

  - Shallow lapse rate, adequate supply of moisture.

- A "mature" thunderstorm has

  - Strong updraft only

  - Strong downdraft only

  - Strong updrafts and downdraft

- Aircraft icing is most favoured in the cloudn regions which have temperature:

  - Ranging between         15 deg C to 40 deg C

  - Ranging between         0 deg C to 15 deg C

  - Below-40 deg C

- Hails is the

  - Solid precipitation which commonly occurs over the mountains regions
during winter.

  - Frozen or partly frozen rain falling from sheet type of clouds

  - Solid precipitation falling from a deep convective cloud.

- Mountain waves are:

  - The waves generated on the windward side of the mountain when there is strong wind flow parallel to the ridge.

  - The waves generated on the leeward side of the mountain when there is a strong wind flow parallel to the ridge.

  - The waves generated on the leeward side of the mountain when there is a strong wind flow perpendicular to the ridge.

- Southwest monsoon period in  the  Indian climatological classification extends from

  - May to September

  - June to October

  - June to September

- Norwesters are

  - The Western disturbances which effect Northwest India

  - Severe thunderstorms which occur over Northeast India during hot weather period

  - Severe thunderstorms which occur over Peninsula during hot weather period

- Fronts are characteristic of:

  - Tropical cyclones

  - Extra Tropical depressions

  - Monsoon Depressions

- In a mature tropical cyclone the Centre and the area 10 - 20 km around it is characterized by

  - Moderate winds and heavy rain

  - Hurricane force winds, fierce squalls and very heavy rain

  - Calm wind, little clouding.

- The tropical cyclones are most frequent over the Indian seas in the:

  - Winter months

  - Post monsoon months

  - Monsoon months

- A monsoon depression is located over the head of the Bay of Bengal South of Calcutta, The most likely surface wind at Calcutta is

  - Westerly 25 Knots

  - Easterly 25 Knots

  - Southerly 25 Knots

- Dust Storms Usually occur over Northwest India during

  - The post – monsoon period

  - The winder period

  - The pre – monsoon period

- Delhi reports rainfall on a day in the month of January. You would infer that the rainfall is associated with

  - A low pressure area moving from East to West in the neighborhood of Delhi.

  - A Western disturbance moving from West to East in the neighborhood of Delhi.

  - The rainfall report might be wrong as Delhi gets rainfall only in the monsoon season.

- In the month of July Calcutta reports surface visibility as 500 meters in the morning synop. You would infer the low visibility due to

  - Fog

  - Heavy rain

  - Low cloud

- AIREP is the code name given to

  - Aircraft Meteorological Reports Originated by Aircraft in Flight.

  - Current Weather Report is transmitted by ATC to the Aircraft in Flight.

  - Meteorological Reports Broadcast for Aircraft Operation.

- The upper air temperature and dew point observations are taken by

  - Pibal Station

  - Radio sonde Station

  - Rawin Station

- Flying at a height of 30,000 from Calcutta to Delhi in the month of January you are most likely to encounter.

  - Very strong head winds

  - Very strong tail winds

  - Moderate southerly winds

- A constant pressure chart is one on which

  - Lines of constant pressure, called isobars, are drawn

  - Lines of constant height of the isobaric surface are drawn

  - Routes along which an aircraft should fly at constant pressure  is routes

- SIGMET is originated by

  - Meteorological Watch Office (MWO)

  - Air Traffic Control (ATC)

  - Flight Information Centre (FIC)

- The thermal wind is

  - The wind that blows because of thermals.

  - The warm wind that blows down the hills on the leeward side

  - The wind which must be added vectorially to the Geostrophic wind at the lower level in order to obtain the Geostrophic wind at the upper level.

- The stratosphere is

  - A layer of the atmosphere where most of the weather activity occurs.

  - The highest layer of the atmosphere

  - An upper atmospheric layer characterized by temperature inversion and stability

  - None the above

- Assuming  the  water vapour  content  of the  atmosphere  to  be  constant throughout on a sunny day, the relative humidity will very as follows:

  - It is maximum in the morning hours, minimum in the afternoon and
then again rises

  - It is minimum in the morning, maximum in the afternoon and then
again falls.

  - Since   there is no change in the water vapour content of the atmosphere, the relative humidity will also not have any variation during the day.

- An air parcel is at a temperature of 30 deg C and a pressure of 1000 Mb. It is cooled till it gets saturated without changing the water vapour content of the parcel and its pressure. The temperature at this stage is observed to be 20 deg C. This temperature of 20 deg C is

  - The potential temperature of the air parcel.

  - The due point temperature

  - The wet bulb temperature

- The air temperature at the surface level is 32 deg C and that at 1.5 Km height above the surface is 33 C as recorded in a particular sounding of the atmosphere. Identity the correct situation(s) from the following:

  - The atmospheric layer surface - 1.5 km level is unstable

  - The layer is stable

  - The layer is conditionally unstable

  - The layer is an isothermal layer.

- A barometer indicates a pressure of about 1000 Mb at a station situated very close mean sea level. The barometer is taken to a height of 3 Km immediately above, it will indicate a pressure of around:

  - 500 Mb

  - 300 Mb

  - 700 Mb

- Gradient wind is the wind:

  - Which flows parallel to the curved isobars

  - Which flows parallel to the straight isobars

  - Which flows across the isobars from high pressure to low pressure
against the" pressure gradient.

- Altostratus is:

  - A low cloud of sheet type

  - A medium cloud of sheet type

  - A cloud of a large vertical growth

  - A high cloud of sheet type

- A high pressure area is usually associated with

  - Fine weather

  - Thunderstorms

  - Strong Winds

- Cold front is normally associated with

  - Cb clouds with frequent precipitation in the form of showers

  - Ci, Cs As & St type of clouds & steady precipitation

  - Fine whether

- Flying at 25,000 feet from Bombay to Delhi on normal day in July, you would expect

  - Easterly winds

  - Westerly winds

  - Northerly Winds

  - Southerly Winds

- During winter season, fog will be more frequent in

  - Assam

  - Rajasthan

  - Bihar

- A Radio Sonde is used to determine

  - The direction and speed of winds at various upper level

  - The temperature and humidity at various levels

  - The height of the clouds over the station

- If the altimeter reads aerodrome elevation at ground level then the subscale must have	been set to

  - QFE

  - QNE

  - QNH

- SIGMENT is issued for

  - Severe line squall

  - Gusty winds

  - Heavy rains

- SPECI is a weather report issued

  - Whenever specially requested for

  - Only when thunderstorm is expected

  - As soon as the specified criteria of a Meteorological condition is satisfied.

- Fill in the blanks choosing the correct words

  - Air can hold more moisture if it is ..................... (Cooled/ heated)

  - Evaporation takes place .................................... (at all times/during day only)

  - Heat is released in the process of ......................... (evaporation / condensation)

  - Local wind which is set up when the air cooled by contact with ground& starts flowing down hill is called ......................... (Anabatic Wind / katabatic Wind)

  - Slow down ward motion or air over large area is called ............ ......................... (Subsidence / Convection)

  - The most variable gas in the atmosphere is   .......................... (Water Vapour / Carbon Dioxide).

- Match the following by choosing one from A and one from B:

  - Norwester				Tamilnadu

  - Clear Air Turbulence		Convergence

  - Western disturbances		Jet Stream

  - NE Monsoon			North Eastern

- Parts of India

  - Upward Motion			NW India

- CAVOK   can   be   used   in  current weather message  when  the  following conditions are satisfied

  - Visibility ………………………………………………….. 10KM

  - No Cloud below ……………..ma and no…………….. 1500

  - No …………………………………………………………… Significant weather

- Match the following in Column 'A' against appropriate entries in Column 'B.

- A						B

  - Thunderstorm				Isohypses

  - Fog						Vis less than 2000m

  - Tropical storms				Super cooled water drops

  - TAF					Heavy Hail

  - SIGMET					First gust

  - AIREP					CNS

  - Airframe icing				Vis less than 1000 m

  - Pressure altitude				Anticycleme

  - PVR					Vertical ascent

  - Subsidence 				Inter

  - Convergence 				Spiral cloud bands

  - Constant pressure charts			Two hour

- In India volmet broadcasts are made by

  - All class I met offices

  - Met watch offices

  - Only Bombay & Calcutta met offices.

- The atmosphere pressure at a point may be defined as

  - The force exerted on a unit area due to the weight of the atmosphere above it

  - Is the force of the atmosphere above the point.

  - Weight of the atmosphere on a point.

- The force on a unit area on the surface of the earth exerted by the weight of	atmosphere, above is called

  - Atmosphere, pressure

  - Isobars

  - Isogonals

- Mark the correct statement.

  - Atmosphere pressure decrease with ht.

  - Atmosphere pressure increases with ht.

  - Atmosphere pressure remains constant irrespective of ht

- The aim. Pressure decreases with ht at the following rate:

  - 1 mb for every 27'

  - 1 mb for every 40'

  - 1 mb for every 30 mtrs

  - 1 mb for 27'mtrs

  - 1 mb for 30'

- i.	The pressure decreases rapidly with the height in

  - Cold air

  - Ward air

- The pressure decreases less rapidly with the ht. In :

  - Cold air

  - Ward air

- The lowest temp, in Tropopause exists over:

  - Pole

  - 45 deg lat

  - Equator

- In a standard atmosphere the lapse rate is

  - 5.6 deg C/km

  - 6.5 deg C/km

  - 1.97 deg C/1 000'

  - 2.98 deg C/1000'

- The rate at which an ascending parcel of dry air cools is called.

  - SALR

  - DALR

- D.A.L.R. Means :

  - The rate at which an ascending parcel of air cools

  - The rate at which temp falls with it

  - The rate at which ascending parcel of saturated air cools.

- S.A.L.R means:

  - The rate at which an ascending parcel of dry air cools.

  - The rate at which temp falls with ht.

  - The rate at which ascending parcel of saturated air cools.

- The D.A.L.R. is

  - 5.4 deg. F, 1000'

  - 2.0 deg C/1000'

  - 2.7 deg C/ 10001

  - 1.5 deg C/10001

  - 5 deg C/Km

  - 10deg C/Km

- The dry air is said to be stable if

  - ELR > DALR

  - ELR = DALR

  - ELR > DALR

  - DALR > SALR

- The saturated air is said to be stable if:

  - ELR  < SALR

  - ELR > DALR

  - ELR = DALR

  - ELR > SALR

- The dry air is said to be instable if:

  - DALR < ELR

  - DALR = SALR

  - DALR = ELR

  - DALR < SALR

- The saturated air is said to be instable if

  - ELR = SALR

  - ELR < SALR

  - ELR > SALR

- If the value of ELR is between DALR 8, SALR the atmosphere said to be:

  - Stable

  - Unstable

  - Conditionally stable

- If the ground level temp on 90 F. What will be the temp of ascending dry parcel of air	after reaching 10000.

  - 36 C

  - 60 F

  - 70 F

- If the atm. Temp on the ground level is 15 C, what will be the temp of ascending parcel1	of saturated air after reaching 5000

  - 7.5 C

  - -7.5 C

  - 0 deg

  - 5 C

- Mark the correct statement

  - Dry air is lighter than moisture.

  - Dry air is heavier than moisture

  - Dry air has greater density than moisture

  - Moist air has greater density than dry air.

  - Moist and dry air have same density.

- Two aircraft with their sub-scale of altimeter set to be same pressure 10.13.2 mbs are	flying at the same actual height. One is flying over cold mass other is flying over warm	air mass

  - Ac over cold air mass has greater indicated altitude.

  - Ac over warm mass has greater indicated altitude.

- AC flying from high to low pressure area, its alt, will read

  - Too high

  - Too low

  - Same height

- AC flying from low to high pressure, altitude, reads

  - Too high

  - Too low

  - Same height

- An altimeter of an a/c under reads - when

  - It flies from high to low pressure.

  - It flies from low to high pressure

- A altimeter of an AC over reads - when

  - It flies from high to low pressure

  - It flies from low to high pressure.

- An altimeter of an aircraft reads too high, if

  - AC flies over warmer air mass.

  - AC flies over colder air mass

- An altimeter of an AC reads too low if

  - AC flies over warmer air mass.

  - AC flies over colder air mass.

- If an aircraft flies over warmer air mass, its alt

  - Over reads

  - Under reads

- If an a/c flies over colder mass, its alt

  - Over reads

  - Under reads

- When QFE is set an altimeter, it reads on landing at an aerodrome

  - Elevation of aerodrome

  - Zero height

- The atmosphere pressure at an aerodrome at any time is

  - QFE

  - QNE

  - QFF

- The atmosphere pressure at the aerodrome reduced to MSL taking into consideration	that ISA atmospheric condition exists between station and

  - QFE

  - QNH

  - QFF

- An altitude in ISA at which the prevailing density exists is called:

  - Pressure altitude

  - Density altitude

  - Flight level

- Pressure altitude may be defined as:

  - An altitude in the stand, atmosphere at which is prevailing pressure occurs.

  - An altitude in the stand, atmosphere a! which the prevailing pressure occurs.

  - An altitude in the stand, atmosphere at which the prevailing density occurs.

- Density altitude may be defined as :

  - An altitude in a stand, atmosphere at which the prevailing pressure occurs.

  - The altitude in the stand, atmosphere at which the prevailing density occurs

  - Surface of constant. Pressure which are related to standard datum of 101 3.2 mbs.

- An altitude in the vicinity of an aerodrome at or below which the vertical	position of an	aircraft is controlled with reference to QNH is

  - Transition altitude

  - Transition level

  - Transition layer

- The lowest level above transition altitude is called

  - Transition altitude

  - Transition layer

  - Transition layer

- A air space between transition altitude and transition level is called

  - Transition altitude

  - Transition layer

  - Transition layer

- The transition altitude may be defined as

  - The altitude in the vicinity of an aerodrome, at or below which the vertical position of a is controlled with reference to QNH

  - The lowest flight level above the transition altitude.

  - The air space between transition altitude and transition level

- The transition level may be defined as

  - The alt. In the vicinity of an aerodrome at or below which the vertical
position, of an is controlled with reference to QNH.

  - The lowest flight level over transition altitude.

  - The air space between transition altitude and transition level.

- The breadth of transition layer is always:

  - 1000’

  - 1500’

  - 500’

- When the visibility is reduced to less than 2000 meters but not less than 1000 meters	due to presence of dust particles in the atmosphere is called.

  - Mist

  - Haze

  - Fog

- When the visibility is reduced to less than 2000 meters but not less than 1000 meters	due to presence of moisture in the atmosphere is called.

  - Mist

  - Haze

  - Fog

- When the visibility is reduced to less than 1000 meters in the atmosphere due to the	presence of water particles:

  - Mist

  - Haze

  - Fog

- Haze may be defined as:

  - When the visibility is reduced less than 2000 meters but not less than1100 meters due to presence of water particulars in the atmosphere.

  - When visibility is reduced to less than 2000 meters but not less than 1000 M due to presence of dust particles in the atm

  - When visibility is reduced less than 1000 m due to presence of water particles in the atm.

- Mist may be defined as :

  - When visibility is reduced to less than 2000 m but not less than 1000 m due to presence of water particles in the atm.

  - When visibility is reduced to less than 2000 m but not less than 1000 m due to presence of dust particles in the atm.

  - When visibility is reduced to less than 1000 m due to presence of water particles the atm.

- Fog may be defined as

  - When visibility is reduced to 2000 m but not less than 1000 m due lo presence of water particles in the atmosphere.

  - When visibility is reduced to 2000 meters.

  - When visibility is reduced to less than 1000 meters due to presence of water particles in the atmosphere.

- R.V.R. is always taken when visibility falls below:

  - 2000 meters

  - 1000 meters

  - 1500 meters

- Radiation fog forms

  - Over land

  - Over sea

  - May from during day

- The formation of radiation fog depends upon

  - Heating of the earth during day.

  - Radiation cooling of earth al night.

- Radiation Fog forms over Northen Part of India during

  - May to June

  - Jan to March

  - Oct to Nov

- Fill in the blanks

  - The radiation for activity increases after the passage of ………….. WD

  - Radiation for is essentially a ………………. Phenomena WINTER

  - The radiation fog forms over ……………. LAND

  - Radiation for forms at …………. Only SUNRISE

  - The advection for forms due to horizontal movement of air ………..

- Advection fog forms:

  - During Night only.

  - During day time only.

  - During any time of day or night.

- The essential conditions of the formation of CB clouds or thunder storm are

  - Steep lapse rate

  - Lapse rate may be below normal

  - Inverted lapse rate

  - Adequate moisture

  - Trigger action to the raise the moist air in the level of instability.

- The trigger action may be take place by following process.

  - Clear night sky no wind

  - Orographic lifting

  - Solar heating during day

  - Divergence associated with high pressure system

  - Convergence associated with low pressure system

  - Frontal lifting

- The life cycle of CB clouds consisting of

  - 2 stages comprising formation, mature and dissipating

  - Only formation stage

  - Only formative and dissipating stages

- The life cycle of CB is usually

  - 30 to 45 min

  - 2 to 3 hrs

  - Only 1 hrs

- The activity of CB cloud is severest for

  - 2 hrs

  - 30 to 45 min

  - 3 to 4 hrs

- Low amorphous cloud layer of dark gray in colour is called

  - AS

  - NS

  - ST

  - CB

  - CU

- A uniform layer of cloud resembling fog but not resting on the ground

  - AS

  - MS

  - ST

  - CB

  - CE

- Icing occurs in

  - CI

  - CS

  - NS

  - AC

  - CE

  - CB

- The cloud, which is composed of ice crystals and is white in colour having feather like	appearance.

  - CI

  - CS

  - NS

  - AC

- The condensations trail occurs in

  - CI

  - CS

  - AO

  - ST

- Drizzle occurs with

  - CI

  - CS

  - AO

  - ST

- Showers occurs with

  - CU

  - CB

  - AB

  - AC

- The most dangerous clouds for aviation are

  - CB

  - CU

  - ST

  - AC

- The air mass which originated from land areas located in the lower latitude are

  - Warm & dry

  - Ward & Moist

  - Cold & Dry

  - Cold & Moist

- The air mass which originates from sea area located in lower Lai. is

  - Warm & Dry

  - Ward & Moist

  - Cold & Dry

  - Cold and Moist

- The air mass which originated from land area located in polar region.

  - Warm & Dry

  - Ward & Moist

  - Cold & Dry

  - Cold and Moist

- The air mass, which originates from land area located in polar region.

  - Polar maritime

  - Polar continental

  - Tropical maritime

  - Tropical continental

  - Equatorial maritime

- The front where the ward air mass over takes the cold air mass, it is called

  - Warm front

  - Cold front

- The front where cold air mass over takes the warm air mass it is called

  - Warm front

  - Cold front

- NS clouds form

  - At cold front

  - At ward front

- The conditions are always unstable at

  - Cold front

  - Warm front

- Precipitation occurs 200 miles ahead of

  - Warm front

  - Cold front

- Precipitation occurs ahead of

  - Warm Front

  - Cold Front

- Precipitation occurs over a belt of 20-30 miles, which extend on both side of front

  - Cold front

  - Warm front

- Mark the correct statement

  - At cold front thunder storm occurs

  - At warm front thunder storm occurs

- CB clouds occurs in

  - Cold front

  - Warm front

- Mark the correct statement

  - Cold front may move as line squall.

  - Warm front may move as line squall

- The ratio between the amount of water vapour present in the given sample of air to the	amount of water vapour that this sample of air can hold at the same temp, is

  - Humidity

  - Relative Humidity

  - Dew point

- The temp in which a given sample of air be cooled at const. Pressure and const water	Vapor, contents in order to become saturated in :

  - Wet bulb temp

  - Dry bulb temp

  - Dew point

  - Rel. Humidity

- Lines drawn through places of equal pressure is known as:

  - Isobars

  - Isotherms

  - Isogonal

  - Isoclinals

- Lines drawn though place having equal temperature is called.

  - Isobars

  - Isotherms

  - Isogonal

  - Isoclinals

- Buy's Bellet law states that if an observer stands with his back to the wind in N	hemisphere.

  - Low exists to his left

  - Low experts to his right

  - High experts to his right

  - High experts to his left.

- If an observer stands with his back to the wind in S-Hemisphere.

  - Low experts to his left

  - Low experts to his right

  - High experts to his right

  - High experts to his left.

- If in N-Hemisphere a/c files from H to L it will experience

  - ‘S’ drift

  - ‘P’ Drift

- If an a flies in S hemisphere from H to L it will experience

  - ‘S’ drift

  - ‘P’ Drift

- If an N-Hemisphere flies from L to H, it will experience

  - ‘S’ drift

  - ‘P’ Drift

- If an ac in S-Hemisphere flies from L to H, II will experience.

  - ‘S’ drift

  - ‘P’ Drift

- Katabatic winds occur

  - During day only

  - At any time of day or night

  - At night only.

- Anabatic winds occur

  - During day only

  - At any time of day or night

  - At night only

- Mark the correct statement.

  - Anabatic wind is stronger than katabatic

  - Katabatic wind is stronger than anabatic

  - Katabatic wind occurs due 10 down ward movements of air along valley

  - Katabatic wind occurs due to upward movement of air along valley

  - Anabatic wind occurs due to downward movement of air along valley

- The wind that blows from sea to land during day (evening)

  - Sea breeze

  - Land breeze

- The wind that blows from land to sea during day night

  - Sea breeze

  - Land breeze

- Mark the correct statement

  - Sea breeze is stronger than land breeze

  - Land breeze is stronger than sea breeze

- Mark the correct statement

  - The wind blow clock around a low in a N – Hemisphere

  - The wind blow clockwise around a low in a S – Hemisphere

  - The wind blow anti clockwise around a low in a N – Hemisphere

  - The wind blow anti clockwise around a low in a S – Hemisphere

- The deep low is known as

  - Cyclone

  - Depression

  - Trough

  - Ridge

- Mark the correct statement

  - Trough has a frontal characteristics

  - The axis of trough is known on line of discontinuity.

  - After crossing the axis of trough winds veer.

  - After crossing the axis of trouble winds back

- The high pressure is known as

  - Anticyclone

  - Trough

  - Col

- Normally fair weather is associated with

  - Low

  - High

- Bad weather is associated with

  - Low

  - High

- A region between two highs and two is know as

  - Trough

  - Ridge

  - Col

- A bad weather may be associated with

  - High

  - Low

  - Col

- The resultant wind that blows under the influence of pressure gradient and geostrophic	force and cyclosetrophic force due curvature of isobars is called

  - Gradient Wind

  - Geostrophic Wind

- Geostrophic wind is

  - The resultant wind that blows under the influence of pressure and Geostrophic force

  - The resultant wind that blows under the influence of pressure, gradient
Geostrophic force and cyclostrophic force due curvature of isobars.

- Units used for measuring alms, pressure are:

  - Lbs / inch

  - MBs

  - Inches of Igs

  - Dynes/Sec/Seo

- Units used for measuring visibility

  - NM

  - SM

  - Kilometers / meters

  - Feet

- The cloud base is reported in

  - Meters

  - Feet

  - Kilometers

  - Miles

- Mark the correct statement

  - DMO's maintain round the clock watch.

  - MMO's maintain round the clock watch

  - SMO's can provide route forecast without prior information.

  - DMO's maintain watch down to dusk.

- During the month of Jan-Feb (winder month) rainfall occurs

  - Over Southern tip of India (Central part of India)

  - Radiation Fog occurs in Southern part of country

  - In Bengal & Bihar

- During the month of Jan-Feb

  - Radiation Fog occurs over Northern & Central part of India

  - Radiation Fog occurs in Southern part of country

  - Activity of Radiation  Fog  increases  after the  passage  of WD  in northern India

- Low humidity & temp is the characteristics of

  - Winter months

  - Hot weather

  - Monsoon

  - Post Monsoon

- Hot weather is

  - Jan - Feb

  - March - May

  - June - Sep

  - Oct. Nov. & Dec.

- During hot weather

  - WD cause T IS or dust storm over Punjab & Raj

  - No WD occur in N-Parts of country

  - Track of WD react Northwards as the season advances

  - The track of WD is Southern most in March

- Norwesters occur during

  - Jan - Feb

  - March - May

  - June - Sep

  - Oct. - Dec.

- Norwesters occur during

  - Winter months

  - Hot weather

  - Monsoon

  - Post Monsoon

- Norwesters occur over

  - Northern India

  - Southern India

  - Central Part of the Country

  - Bengal and Bihar, Orissa and Assam

- During hot weather TRS may occur over

  - Madras

  - Ocean Plato

  - Southern tip of India

  - Bay of Bengal

- TRS occurs over India seas

  - Equatorial Region

  - 10-14deg N

  - 5 - 8 deg N

- Norwesters are

  - Weaker thunder storms

  - Violent thunder storms

  - Similar to cyclones but less active

  - The degree of aviation hazards in the Norwesters, is higher than in a
cyclone

- Norwesters normally occur during

  - Morning hours

  - Afternoon

  - Night hours

- The average life of a Norwesters is

  - 8-9 hours

  - 2-3 hours

  - 7 hours

- Monsoon period occurs

  - Jan to Feb

  - March to May

  - Jun to Sep

  - Oct to Dec

- The rainfall over India during monsoon depends on

  - Higher pressure in Afghanistan

  - Depression that from Bay of Bengal

  - The position of the axis of trough

- The rainfall all over the country occur during the monsoon when

  - Axis of trough of low pressure is in the southern — most

  - Axis of trough of low pressure is along Himalayas

  - Depression as formed over Bay of Bengal.

- It is known to be break in monsoon when

  - Axis of trough of low pressure is in the southern — most

  - Axis of trough of low pressure is along Himalayas

  - Depression as formed over Bay of Bengal.

- In India the monsoon currents are

  - South - westerly current

  - SE ly current

  - NE ly current

  - Both a & c are correct

- The monsoon advances with:

  - Bay of Bengal currents only

  - Arabian sea currents only

  - Advances as two currents. Bay of Bengal and Arabian sea currents

- The SE by trade winds in Southern - Hemisphere are drawn in as S moist	currents in N-Hemisphere under the influence of seasonal low over NW India and	these currents are known as

  - Monsoon

  - Trade winds

  - Cyclone winds

- Maximum w/v in a monsoon low moving from Bay of Bengal along axis of trough occur,

  - NE of Low

  - SW Sector of low

  - NW Sector of low

  - SE Sector of low

- Post monsoon weather occurs.

  - Jan - Feb

  - March - May

  - June – September

  - Oct - Dec

- During post monsoon weather lower pressure occurs over:

  - Bay of Bengal

  - NW India

  - NE India

  - Central India

- During post monsoon higher pressure occurs over

  - Bay of Bengal

  - NW India

  - NE India

  - Central India

- Post monsoon weather is also known as

  - NE Monsoon

  - SW Monsoon

  - SE Monsoon

  - Retreating Monson Season

- Monsoon weather may also be known as

  - NE Monsoon weather

  - SW Monsoon

  - SE Monsoon

- Norwesters originate from

  - Chota - Nagpur hills

  - Dacan plato

  - Sub-mountain district

  - Khasi hills

- Summer season the wind of western coast during day is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer season the wind on Western coast during night is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer season the wind on Eastern coast during day is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- During summer the wind on Eastern coast during night is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer the wind at Bombay during day is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer the wind at Bombay during night is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer the wind at Madras during day is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- In summer the wind at Madras during night is

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- An air mass becomes unstable when

  - Easterly

  - Westerly

  - Northerly

  - Southerly

- Squall are distinguished from gusts by air

  - Shorter duration

  - Longer duration

  - Lower wind speed

- Fronts are found in

  - A thermal low

  - A mid latitude low pressure area

  - A tropical low pressure area

- The high pressure area are generally associated with winds which

  - Back and increases

  - Veers and decreases

  - A tropical low pressure area

- When a cold front passes, the temperature

  - Will suddenly rise

  - Will suddenly fall

  - Will remain constant

- When flying in a stable air mass you can expect

  - Commulous types of slouds

  - Stratus types of cloud with smooth flying condition

  - Stratus type of cloud with turbulence

- The atm. Pressure at a specific level may be defined as:

  - The pressure exerted by one cubic foot of air at that level.

  - The downward force per unit area exerted by air at that level.

  - The density of air at that level.

- The warm front may be defined as

  - The leading edge of an advancing cold air

  - The leading edge of an advancing ward air

  - The ridge of high pressure.

- When sub scale of an altimeter is set to 1013.2 mbs the ht indicated on the altimeter:

  - Is the highest over sea level

  - Is the highest over the aerodrome level

  - Is the flight level.

- Tropical revolving storm occur

  - In the Bay of Bengal in Oct. / Nov.

  - In the Arabian sea in Oct. / Nov.

  - In the North West of India in May / June

- While avoiding the centre of a cyclone in the Northern Hemisphere advantage can be	taken of tail winds by flying to the

  - Left

  - Right

  - North of centre when approaching from any direction

- With the sub scale set to 1013.2 mbs altimeter indicates

  - True altitude a.m.s.1

  - Height above ground

  - Pressure altitude

- If the QNH is higher than 1013.2 mbs the altitude of the Flight level would be

  - Same as standard

  - Less than standard

  - More than standard

- Pressure altitude at a place when QNH is above 1013.2 mbs is

  - More than true altitude

  - Less than true altitude

  - Same as true altitude

- A change of pressure of 1 mb in vertical direction at 6 kms a.m.s.c. would	correspond be a vertical distance of

  - 30 ft

  - Less than 30 Ft

  - More than 30 Ft.

- If the QNH of a station decreases, heights of different fit level above the station.

  - Decreases

  - Increases

  - Remain constants

- The pressure altitude of a place depends only on law atm.

  - Pressure

  - Temp

  - An density

- The higher the elevation of a station the difference between QNH. QFE of the station will	be

  - Greater

  - Less

- An increase of 1000 ft in elevation near the earth is associated with decrease of	pressure of

  - 100 mb

  - 1000 mb

  - 3 mb

  - 33 mb

- Diurnal pressure variation are most pronounced in

  - Polar region

  - Middle latitude

  - Tropic areas.

- The average height of Tropopause over equatorial regions

  - 8 kms

  - 10 kms

  - 16 kms

- A smooth flight may be expected if the radio-sonde observations show

  - Temperature inversion

  - Lapse rate termed as dry adiabatic

  - Super adiabatic lapse rate

- If an a/c flies at a constant indicated  altitude towards an area of over temperature, the	altimeter will

  - Under reads

  - Over reads

  - No change

- In SH around a low pressure area winds blow

  - In a clock wise direction

  - In an anticlockwise direction

  - Across the isobars towards the centre

- In the NH the direction of the thermal wind is

  - At right angles to the isotherms from high to low temp

  - Along the isotherms with low temperature to the left.

  - Along the isotherms with low temperature to the right.

- The appearance of lenticular clouds enables the pilot to infer the presence of

  - Warm front

  - Cold front

  - Standing waves over mountain regions

- A fog formed by warm moist air flowing over a cold surface is called.

  - Advection fog

  - Radiation fog

  - Frontal fog.

- To avoid a/c. icing in flight in cloudy condition a pilot is advised to be guided by.	Optical	phenomenon & fly through a cloud which is seen to have

  - Halo

  - Carona

  - A system of coloured rings called "glory"

- In the Bay of Bengal during the months of July & August tropical depression form.

  - At the head of the bay

  - Central bay

  - South bay

- In India Easterly jet stream is found south bay

  - In the winter months along a lat of 2600n at 12 km

  - During S/W monsoon season at 1 2 km. Near lat 1 300 N.

- During the month of May & June, in India the seasonal low-pressure area lies over.

  - NE India

  - NW India

  - South Bay of Bengal

- During S/W monsoon season the upper winds at 40000 ft over south India are

  - Easterly

  - Westerly

  - South Westerly

- Cyclonic storms cross Tamilnadu coast in

  - October to December

  - May to July

  - February to May

- The surface discontinuity between Polar Easterlies & the temperature Westerlies is	called.

  - Equatorial front

  - Tropopause

  - Polar front.

- Westerly disturbances affect the

  - the southern part of India from November to February

  - Northern part of India from December to March

  - Western part of India from July to September