- METEROLOGY

- The International standard atmosphere assumes a lapse rate of

  - 2oC/1000 ft.

  - 1.5oC/1000ft.

  - 3oC/1000 ft.

  - 1.98oC/1000 ft.

- The tropopause is:

  - The line where the temperature no longer decreases with increase of height.

  - The layer between the tropopause and the stratosphere.

  - The layer beyond which only CI cloud occurs.

  - The line indicating clear air turbulence.

- One of the most important characteristics of the atmosphere is:

  - Density is constant above 10,000 ft.

  - The air is a poor conductor of heat.

  - Temperature lapse rate is very frequently above 3oC per 1000 ft.

  - The air is a good conductor of heat.

- Most of the vapour in the atmosphere is contained in the:

  - Tropopause.

  - Stratosphere.

  - Troposphere.

  - Stratopause.

- The captain of an aircraft needs to know the height of the Tropopause because:

  - It normally represents the limit of weather.

  - Density starts to increase.

  - There are no longer jet streams and CAT.

  - It indicates the heights of the thermal wind.

- The main Ozone layer is to be found in the:

  - Thermosphere

  - Troposphere

  - Mesosphere

  - stratosphere

- The level in the atmosphere where the air temperature ceases to fall with increase in height is known as:

  - The troposphere.

  - The Stratopause.

  - The Stratosphere.

  - The tropopause.

- Which statement is correct when considering the lower layers of the atmosphere:

  - The majority of the weather is contained in the stratosphere and its upper boundary is the tropopause.

  - The majority of the weather is contained in the troposphere and its upper boundary is the tropopause.

  - The majority of the weather is contained in the tropopause and its upper boundary is the tropopause.

  - The majority of the weather is contained in the troposphere and its upper boundary is the stratosphere.

- The atmosphere is a mixture of gasses of the following proportions:

  - Oxygen 21%		nitrogen 78%		other gasses 1%.

  - Oxygen 21%		hydrogen 78%		other gasses 1%.

  - Nitrogen 78%		argon 21%		oxygen  1%.

  - Nitrogen 78%		oxygen 21%		hydrogen  1%.

- In the ISA the temperature is isothermal:

  - Up to 36 090 ft/11 kms.

  - From 36 090 ft/11 kms to 65 617 ft/20 kms.

  - From 36 090 ft/11 kms to 104 987 ft/32 kms.

  - From 36 090 ft/11 kms to 45 090 ft/13.75 kms.

- The International ( ICAO) Standard Atmosphere assumes that the sea level atmospheric pressure is:

  - 1013.25 mbs and decreases with an increase in height.

  - 1013.25 mbs and increases with an increase in height.

  - 1013.25 mbs and falls to about half this value at 30000.

  - 1013.25 mbs and decreases with an increase in height up to the tropopause. Above the tropopause it remains constant.

- At  sea level the ISA density is stated to be:

  - 1225 grammes per cubic metre.

  - 1252 grammes per cubic metre.

  - 1013.2 mb ( hpA)

  - 29.6 inches of mercury.

- Which of the following statements is most correct when describing ISA:

  - The MSL pressure is 1013.25 mbs and the temperature is + 15oC.

  - The MSL pressure is 1013.25 mbs and the temperature is + 15oC with a lapse rate of 1.98oC/1000 ft.

  - The MSL pressure is 1013.25 mbs and the temperature is + 15oC with a lapse rate of 1.98oC/1000 ft up to 36090 ft above which there is frequently an inversion.

  - The MSL pressure is 1013.25 mbs and the temperature is + 15oC with a lapse rate of 1.98oC/1000 ft up to 36090 ft.

- The following is true for the International Standard Atmosphere:

  - At mean sea level the following conditions prevail: temperature + 15 C, pressure 1013.25 hpa, density 1125 gm/m.

  - Within the troposphere the temperature decreases by 6.5 C per km.

  - The tropopause is at a height of 36090 AGL.

  - The temperature at the tropopause is 226.5oK.

- Pressure will _________ with increase of height and in the ISA pressure will be ________ at 10 000 ft and _______ at 30 000 ft:

  - Increase		800 mb 400 mb.

  - Decrease		700 mb 300 mb.

  - Increase		200 mb 800 mb.

  - Decrease		500 mb 200.

- PRESSURE

- The barometric Pressure at the airfield datum point is known as:

  - QNE.

  - QNH.

  - QFE.

  - Standard Pressure.

- The instrument that gives a continuous printed reading and record of the atmospheric pressure is:

  - Barometer.

  - Hygrometer.

  - Anemograph.

  - Barograph.

- The pressure of the atmosphere:

  - Decreases at an increasing rate as height increases.

  - Decreases at a constant rate as height increases.

  - Decreases as a decreasing rate as height increases.

  - Decreases at a constant rate up to the tropopause and then remains constant.

- When considering the actual tropopause which statement is correct:

  - It is low over the poles and high over the equator.

  - It is high over the poles and low over the equator.

  - It is the same height of 36090 ft all over the world.

  - It is at a constant altitude of 26000’.

- Atmospheric pressure may be defined as:

  - The weight of the atmosphere exerted on any surface with which it is in contact

  - The weight of the atmosphere at standard sea level.

  - The force per unit area exerted by the atmosphere on any surface with which it is in contact.

  - A pressure exerted by the atmosphere of 1013.2 mbs.

- The QFE is the atmospheric pressure:

  - At the place where the reading is taken.

  - Corrected for temperature difference from standard and adjusted to MSL assuming standard atmospheric conditions exist.

  - At a place where the reading is taken corrected to MSL taking into account the prevailing temperature conditions.

  - As measured by a barometer at the aerodrome reference point.

- With 1013.25 mb set on the altimeter sub scale with an aircraft stationary on the airfield the altimeter will read :

  - QNE.

  - QNH.

  - QFE.

  - QFF.

- The aircraft altimeter will read zero at aerodrome level with which pressure setting set on the altimeter sub scale:

  - QFE.

  - QNH.

  - QNE.

  - QFE.

- You are passed an altimeter setting of ‘ 29.53’. You would then set your altimeter subscale to:

  - QFE.

  - 1013.

  - 1000.

  - QFE.

- The aerodrome QFE is:

  - The reading on the altimeter on an aerodrome when the aerodrome barometric pressure is set on the sub scale.

  - The reading on the altimeter on touchdown at an aerodrome when 1013.2 is set on the sub scale.

  - The reading on the altimeter on an aerodrome when the sea level barometric pressure is  set on the sub scale.

  - The  aerodrome barometric pressure.

- When an altimeter sub scale is set to the aerodrome QFE, the altimeter reads:

  - The elevation of the aerodrome at the aerodrome reference point.

  - Zero at the aerodrome reference point.

  - The pressure altitude at the aerodrome reference point.

  - The appropriate altitude of the aircraft.

- The aerodrome QNH is the aerodrome barometric pressure :

  - Corrected to mean sea level assuming standard atmospheric conditions exist.

  - Corrected to mean sea level assuming isothermal  conditions exist.

  - Corrected for temperature and adjusted to MSL assuming standard atmosphere conditions exist.

  - Corrected to MSL using ambient temperature.

- A line drawn on a chart joining places having the same barometric pressure at the same level and at the same time is :

  - An isotherm.

  - An isallobar.

  - A contour.

  - An isobar.

- An isobar on a meteorological chart joins all places having the same:

  - QFE.

  - QFF.

  - QNH.

  - QNE.

- Pressure will ----------- with increase of height and will be about --------- at 10000 ft and ------ ----at 30000 ft.

  - Increase		800 mb		400 mb.

  - Decrease		700 mb		300 mb.

  - Increase		200 mb		800 mb.

  - Decrease		500 mb		200 mb.

- An airfield in England is 100 m above sea level, QFE is 1030hPa, temperature at the surface is -15oC. What is the value of QNH?

  - Impossible to determine.

  - Less than 1030 hPa.

  - Same as QFF.

  - More than 1030 hPa.

- DENSITY

- Consider the following statements relative to Air Density and select the one which is correct:

  - Because air density increases with decrease of temperature, air density must increase with increase of height in the International Standard Atmosphere ( IAS).

  - At any given surface temperature the air density will be greater in anticyclonic conditions than it will be when the MSL pressure is lower.

  - Air density increases with increase of relative humidity.

  - The effect of change of temperature on the air density is much greater than the effect of change of atmospheric pressure.

- The tropopause in mid latitudes is:

  - Lower in summer with a lower temperature.

  - Lower in winter with a higher temperature.

  - Lower in summer with a higher temperature.

  - Lower in winter with a lower temperature.

- Generally as altitude increases:

  - Temperature decreases and density increases.

  - Temperature, pressure and density decreases.

  - Temperature and pressure increase and density decreases.

  - Temperature decreases and pressure density increases.

- In the troposphere:

  - Over cold air, the pressure is higher at upper levels than at similar levels over warm air.

  - Over cold air, the pressure is lower at upper levels than at similar levels over warm air.

  - Over warm air, the pressure is lower at upper levels than at similar levels over warm air.

  - The upper level pressure depends solely on the relative humidity below.

- Density at the surface will be low when:

  - Pressure is high and temperature is high.

  - Pressure is high and temperature is low.

  - Pressure is low and temperature is low.

  - Pressure is low and temperature is high.

- PRESSURE SYSTEMS

- A trough of low pressure is generally associated with:

  - Convergence causing increased cloud and precipitation.

  - Divergence causing increased cloud and precipitation.

  - Subsidence causing increased cloud and precipitation.

  - Subsidence causing decreased cloud and precipitation.

- A ridge of high pressure is generally associated with:

  - Convergence causing increased cloud and precipitation.

  - Divergence causing increased cloud and precipitation.

  - Divergence causing cloud to break up and more precipitation.

  - Divergence and subsidence causing clear skies  and good weather.

- A small low established within the circulation of another low is called

  - A trough.

  - A col.

  - An anticyclone.

  - A secondary depression.

- An area of indeterminate pressure between two lows and two highs is called:

  - A trough.

  - A ridge.

  - A col.

  - A saddle.

- A trough of low pressure is:

  - A small low established within the circulation of another low.

  - An extension or elongation of a low pressure system along an axis on each side of which pressure increases.

  - A centre of pressure surrounded on all sides by higher pressure.

  - An area where the pressure is lower than anywhere else in the area.

- If in the southern hemisphere an aircraft in flight at 2000 ft. is experiencing starboard drift, the aircraft is flying towards.

  - An area of high pressure.

  - An area of low pressure.

  - A warm front.

  - A depression.

- In the Southern Hemisphere, the surface winds at B1; and C2 would be respectively:

  - Clockwise across the isobars away from the centre: and anti-clockwise across the isobars towards the centre.

  - Anti- clockwise across the isobars towards the centre: and clockwise across the isobars away from the centre.

  - Anti- clockwise across the isobars away from the centre: and clockwise across the isobars towards the centre.

  - Clockwise across the isobars towards the centre: and Anti-clockwise across the isobars away from the centre.

- Subsidence in an anticyclone produces:

  - Saturated air and an inversion.

  - Dry air and an inversion.

  - Isothermal dry and stable air.

  - Increased pressure at the surface.

- With an anticyclone over the UK the expected weather is:

  - Thunderstorms in summer, fog in winter.

  - Stratus in summer with drizzle, CU and snow in winter.

  - Clear skies or fair weather CU in summer, fog in winter.

  - Clear skies in summer with haze, cold frontal weather in winter.

- Refer to appendix A and answer questions 10 to 14

- The pressure systems at A2; B1; B2; B3 and C2 are respectively:

  - Depression; anticyclone; Col; Ridge; and Trough.

  - Ridge; Anticyclone; Col; Trough; and Depression.

  - Trough; Depression; Col; Ridge; and Anticyclone.

  - Ridge; Depression; Col; Trough; and Anticyclone.

- Two important weather factors at B2 will be:

  - Frontal weather in winter, fog in summer.

  - Clear conditions in summer, thunderstorms in winter.

  - Thunderstorms in summer, fog in winter.

  - Fog in summer, thunderstorms in winter.

- Haze in summer and radiation fog in winter can be expected at:

  - C2.

  - B3.

  - B1.

  - B2.

- In the non- frontal pressure system at B3, the expected weather:

  - ST SC with drizzle or light precipitation.

  - Clear skies with moderate winds.

  - CU CB with showers.

  - Light winds and haze with an inversion.

- ALTIMETRY

- An aircraft is flying at 3000 feet indicated with the altimeter sub scale set to 1020 mb towards a mountain range with an elevation of 1600 feet. If during the flight the QNH in the area falls to 989 mb and the altimeter sub scale is not reset, the expected clearance over the mountain range will be: ( assume 27 feet = 1 mb)

  - 1400 ft.

  - 470 ft.

  - 930 ft.

  - 563 ft.

- When flying towards a depression at a constant indicated altitude, the true altitude will be:

  - Lower than indicated.

  - Higher than indicated.

  - The same as indicated.

  - Lower than indicated at first then the same as indicated later.

- The name given to the lowest forecast mean sea level pressure in an area is:

  - QFE.

  - Regional QNH.

  - QFF.

  - QNE.

- The Altimeter will always read:

  - With 1013 set the altitude above MSL.

  - With airfield QNH set the height above the airfield datum

  - The vertical distance above the pressure level set.

  - The correct flight level with regional QFE set.

- An aircraft at airfield P elevation 270 ft has the airfield QNH 1012 mbs correctly set. The altimeter setting is not changed. Later on landing at airfield Q elevation 450 ft the aircraft altimeter reads 531 ft. What is the correct QNH at airfield Q? ( Assume 27 ft= 1 mb)

  - 1014.7 mb

  - 1009.3 mbs.

  - 1015 mbs.

  - 1009 mbs.

- The altimeter subscale is set to 1030 mbs and altimeter reads 4500. QNH is 996 mbs. What is the altitude of the aircraft? ( Assume 1 mb = 27’)

  - 3480’

  - 3990’

  - 5418’

  - 3582’

- An aircraft flies level with a mountain peak shown on a chart as 8000’ft above MSL. If the temperature of the air is ISA +2, what will be indicated on the pressure altimeter?

  - 8064 ft.

  - 7936 ft.

  - 8000 ft.

  - 7968 ft.

- An aircraft, flying at FL 100 at a constant RAS, flies from an area of warm air into an area of cold air. The QNH is unchanged. How has the aircraft altitude and TAS changed?

- Altitude		TAS

  - Decreased		increased.

  - Increased		increased.

  - Decreased		decreased.

  - Increased		decreased.

- On a flight from Capetown ( QNH 1005 hPa) to Dakar ( QNH 1018 hPa) at FL 240 you notice a decrease in true altitude. Why might this be the reason?

  - It is warmer in Dakar than in Capetown.

  - It is colder in Capetown than in Dakar.

  - The static vent is blocked.

  - It is colder in Dakar than in Capetown.

- QNH at Johannesburg is f1025 hPa, elevation is 1600 m amsl. What is the QFE. ( Assume 1 mb= 8m)

  - 1000.8 hPa.

  - 830.6 hPa.

  - 1002 hPa.

  - 825 hPa.

- When flying from Paris (QNH 1012) to London ( QNH 1015) at FL 100. You neglect to reset your altimeter but why does your true altitude remain the same throughout the flight.

  - Paris has a higher pressure than London.

  - The air at London is warmer than Paris.

  - London is at a lower altitude than Paris.

  - The air at Paris is warmer than London.

- An airfield in Holland is 20m below sea level, QFE is 1020 hPa, temperature at the surface is +30oC. What is the value of QNH.

  - Impossible to determine.

  - Less than 1020 hPa.

  - Same as QFF.

  - More than 1020 hPa.

- The measurement of surface temperature is made:

  - At ground level.

  - At approximately 10 meters from ground level.

  - At approximately 4 feet above  ground level.

  - At approximately 4 meters above ground level.

- The purpose of a “Stevenson Screen” is to:

  - Maintain a moist atmosphere so that the wet bulb thermometer can function correctly.

  - To prevent the mercury freezing in the low winter temperatures.

  - Protect the thermometer from wind, weather and from direct sunshine.

  - Keep the wet and dry bulb thermometers away from surface extremes of temperature.

- If temperature remains constant with an increase in altitude there is:

  - An inversion.

  - An inversion aloft.

  - Uniform lapse rate.

  - An isothermal layer.

- The surface of the earth is heated by:

  - Convection.

  - Conduction.

  - Long wave solar radiation.

  - Short wave solar radiation.

- Cloud cover will reduce diurnal variation of temperature because:

  - Incoming solar radiation is reflected back to space and outgoing terrestrial radiation is reflected back to earth.

  - Incoming solar radiation is re-radiated back to space and atmospheric heating by convection will stop at the level of the cloud layer.

  - The cloud stops the suns rays getting through to the earth and also reduces outgoing conduction.

  - Incoming solar radiation is reflected back to space and outgoing terrestrial radiation is re-radiated from the cloud layer back to the surface.

- Diurnal variation of the surface temperature will:

  - Be unaffected by a change of wind speed.

  - Decrease as wind speed increases.

  - Increase as wind speed increases.

  - Be at a minimum in calm conditions.

- Which of the following surfaces is likely to produce a higher than average diurnal variation of temperature:

  - Rock or concrete

  - Water.

  - Snow.

  - Vegetation.

- Most accurate temperatures above ground level are obtained by:

  - Tephigram.

  - Aircraft reports.

  - Temperature probe.

  - Radio sonde.

- The method by which energy is transferred from one body to another by contact is called:

  - radiation

  - convection.

  - Conduction.

  - Latent heat.

- The diurnal variation of temperature is:

  - Greater over the sea than overland.

  - Less over desert areas then over temperate grassland.

  - Reduced anywhere by the presence of cloud.

  - Increased anywhere as wind speed increases.

- The troposphere is heated largely by:

  - Absorption of the sun’s short wave radiation.

  - Radiation of heat from cloud tops and the earth’s surface.

  - Absorption by ozone of the sun’s short wave radiation.

  - Conduction from the surface, convection and the release of latent heat.

- An inversion is one in which:

  - There is no horizontal gradient of temperature.

  - There is no change of temperature with height.

  - There is an increase of temperature as height increases.

  - There is a decrease of temperature as height increases.

- The sun gives out ________ amount of energy with _________ wavelengths. The earth gives out relatively __________ amounts of energy with relatively ________ wavelengths:

  - Large, large, small, small.

  - Small, small, large, large.

  - Large, large, small, large.

  - Large, small, small, large.

- With a clear night sky, the temperature change with height by early morning is most likely to show:

  - A steady lapse rate averaging 2oC per 1000 ft.

  - A stable lapse rate of 1oC per 1000 ft.

  - An inversion above the surface with an isothermal layer above.

  - An inversion from near the surface and a 2oC per 1000 ft. lapse rate above.

- Over continents and oceans, the relative temperature conditions are:

  - Warmer in winter over land, colder in summer over sea.

  - Colder in winter over land, warmer in winter over sea.

  - Cold in winter over land and sea.

  - Warmer in summer over land and sea.

- HUMIDITY

- Throughout the 24 hrs. of a day the Relative Humidity can be expected to:

  - Increase during the day and decrease at night.

  - Stay reasonably constant throughout the 24 hours.

  - Reduce during the day and increase at night.

  - Only change with a change of airmass.

- During a night with a clear sky, surface temperature will __________ RH will ___________ and Dew point will _________

  - Fall, Rise, Rise.

  - Rise, Rise, Fall.

  - Fall, Rise, Remain, the same.

  - Fall, Fall, Remain the same.

- A change of state directly from a solid to a vapour or vice versa is:

  - Insolation.

  - Condensation.

  - Evaporation.

  - Sublimation.

- The instrument used for measuring the humidity of air is a :

  - Hydrometer.

  - Hygrometer.

  - Wet bulb thermometer.

  - Hygroscope.

- The process of change of state from a gas to a liquid is:

  - Evaporation in which latent heat is absorbed.

  - Evaporation in which latent heat is released.

  - Condensation in which latent heat is absorbed.

  - Condensation in which latent heat is released.

- The process of change of state from a liquid to a gas is:

  - Condensation in which latent heat is released.

  - Evaporation in which latent heat is released.

  - Condensation in which latent heat is absorbed.

  - Evaporation in which latent heat is absorbed.

- Air is classified as dry or saturated according to its relative humidity. If the relative humidity were 95% the air would be classified as:

  - Conditionally saturated.

  - Partially saturated.

  - Saturated.

  - Dry.

- On  a wet bulb thermometer in an unsaturated atmosphere there will be a reduction of temperature below that of the dry bulb thermometer because:

  - Heat is absorbed during the process of condensation.

  - Heat is released during the process of condensation.

  - Heat is absorbed by the thermometer during the process of evaporation.

  - Heat is released from the thermometer during the process of evaporation.

- Relative humidity is:

  - Air temperature over wet bulb temperature x 100.

  - Air temperature over dew point temperature x 100.

  - The actual amount of water vapour in a sample of air over the maximum amount of water vapour that the sample can contain x 100

  - The maximum amount of water vapour that a sample of air can contain over the actual amount of water vapour the sample does contain x 100.

- Absolute humidity is :

  - The number of water droplets in a given quantity of air.

  - The amount of water vapour that a given quantity of air holds.

  - The maximum amount of water vapour that a given quantity of air can hold.

  - The maximum number of water droplets that a given quantity of air can hold.

- Wet bulb temperature would normally be lower than the dry bulb temperature because:

  - Condensation causes a release of latent heat.

  - Evaporation causes cooling.

  - Latent heat is absorbed by the bulb thermometer.

  - Of condensation on the muslin wick of the bulb

- The wet bulb temperature:

  - Is measured using a hydrometer.

  - Is the minimum temperature to which a thermometer bulb can be cooled by the evaporation of water.

  - Measures the dew point of the air.

  - Is the minimum temperature reached by the surface of the earth as measured by a thermometer placed 1.2 metres above the ground.

- Which one of the following statements relating to atmospheric humidity is correct:

  - If the air temperature falls then the absolute humidity must increase.

  - The absolute humidity is the mass of water vapour contained in unit volume of air.

  - The diurnal variation of dew point temperature is greatest when skies are clear at night.

  - The dew point temperature is the temperature indicated by the wet bulb thermometer.

- When condensation takes place, the higher the temperature, the ________ the amount of latent heat _________ :

  - Lesser; released.

  - Greater; absorbed.

  - Greater ; released.

  - Lesser ; absorbed.

- When water vapour changes to ice:

  - Latent heat is absorbed.

  - Specific heat is released.

  - Latent heat is released.

  - Specific heat is absorbed.

- ADIABATICS AND STABILITY

- If the ELR is 0.65oC/100m

  - Atmosphere is conditionally stable.

  - Atmosphere is  stable.

  - Atmosphere is unstable.

  - Atmosphere is  stable when dry.

- ELR is 1oC/100m

  - Neutral when dry.

  - Absolute stability.

  - Absolute instability.

  - Conditional stability.

- Why does air cool as it rises?

  - It expands.

  - It contracts.

  - The air is colder at higher latitudes.

  - The air is colder at higher altitudes.

- From which of the following can the stability of the atmosphere be determined?

  - Surface pressure.

  - Surface temperature.

  - DALR.

  - ELR.

- When the upper part of a layer of warm air is advected:

  - Stability increases within the layer.

  - Stability decreases within the layer.

  - Wind speed will always decrease with increase in height in the Northern Hemisphere.

  - Wind will back with increase in height in the Northern Hemisphere.

- The temperature at the surface is 15oC, the temperature at 1000m is 13oC. Is the atmosphere

  - Unstable.

  - Conditionally unstable.

  - Stable.

  - Cannot tell.

- Which of the following gives conditionally unstable conditions?

  - 1oC/100m.

  - 0.65oC/100m.

  - 0.49oC/100m.

  - None of the above.

- A mass of unsaturated air is forced to rise till just under the condensation level, It then settles back to its original position.

  - Temp. is greater than before.

  - Temp. stays the same.

  - Temp. is less than before.

  - It depends on QFE.

- What happens to the stability of the atmosphere in an inversion? ( Temp increasing with height)

  - Absolutely stable.

  - Unstable.

  - Conditionally stable.

  - Conditionally unstable.

- What happens to stability of the atmosphere in an isothermal layer ? ( Temp constant with height)

  - Absolutely stable.

  - Unstable.

  - Conditionally stable.

  - Conditionally unstable.

- What is the effect of a strong low level inversion?

  - Good visibility.

  - Calm conditions.

  - Turbulence.

  - Unstable conditions.

- A layer of air can be:

  - Conditional; unstable when unsaturated and stable when saturated.

  - Conditional; unstable when saturated and stable when unsaturated.

  - Neutrally stable when saturated and unstable when unsaturated.

  - All of the above.

- What happens to the temperature of a saturated air mass when forced to descend?

  - It heats up more than dry because of expansion.

  - It heats up less than dry because of evaporation.

  - It heats up more than dry because of sublimation.

  - It heats up less than dry because of latent heat released during condensation.

- In still air  a lapse rate of 1.2oC/100 m refers to:

  - DALR.

  - SALR.

  - ELR.

  - ALR.

- What happens to the temperature of a saturated air mass when descending?

  - It heats up more than dry because of expansion.

  - It heats up less than dry because of evaporation.

  - It heats up more than dry because of compression.

  - It heats up less than dry because of latent heat released during condensation.

- The DALR is

  - Variable with time.

  - Fixed.

  - Variable with latitude.

  - Variable with temperature.

- A parcel of air cooling by more than 1oC/100m is said to be?

  - Conditionally stable.

  - Conditionally unstable.

  - Unstable.

  - Stable.

- Maximum turbulence associated with the standing waves is likely to be:

  - Two wavelengths down wind and just above the surface.

  - Approximately one wavelength down wind of, and approximately level with, the top of the ridge.

  - Just below the tropopause above the ridge.

  - Down the lee side of the ridge and along the surface.

- For the formation of mountain waves, the wind above the level of the ridge should:

  - Decrease or even reverse direction.

  - Increase initially then decrease.

  - Increase with little change in direction.

  - Increase and then reverse in direction.

- When flying in IMC in a region close to a range of hills 2000 ft. high in stable air and with wind direction at right angles to the axis of the range of hills, which of the following is probably the most dangerous practice:

  - Flying towards the hills, into the wind, at flight level 65.

  - Flying parallel to the hills on the downwind side at flight level 40.

  - Flying towards the hills downwind at flight level 55.

  - Flying parallel to the hills on the upwind side at flight level 40.

- Which of the following statements referring to Jet Streams is correct:

  - CAT associated with Jet Streams is probably associated with the rapid wind shear in the vicinity of the jet.

  - The maximum windspeed in a Jet Stream increases with increase of height up to the tropopause and remains constant thereafter.

  - The core of a Jet Stream is usually located just below the tropopause in the colder air mass.

  - The rate of change of windspeed at any given level is usually greatest on the warmer side of the jet.

- Note --- Question No 5 to 8 not feed in OMR

- The wind at square A3 is likely to be:

  - 35 kt.

  - 50 kt.

  - 25 kt.

  - Light.

- The wind at ABC 4 is likely to be:

  - 50 kt.

  - 40 kt.

  - 35 kt.

  - A jet stream.

- Flight conditions at B1 are likely to be:

  - Smooth.

  - Turbulent.

  - Turbulent in breaking wave crests.

  - Turbulent due to marked up and down currents.

- The most extreme turbulence can occur:

  - At B1.

  - At A2.

  - At ABC 4.

  - At B2, 3, 4 and at C2, 3, 4.

- The significance of lenticular cloud is:

  - There may be mountain waves present and there will be severe turbulence.

  - There are mountain waves present but they may not give severe turbulence.

  - A Fohn wind can be expected with no turbulence.

  - A Katabatic wind is present which may lead to fog in the valleys.

- A mountain range is aligned in an East/West direction. Select the conditions from the table below that will give rise to Standing Waves:

- 2000 ft	5000 ft.		10 000 ft.

  - 020/40	020/30		020/50

  - 170/20	190/40		210/60

  - 270/15	270/20		270/40

  - 090/20	090/40		090/60

- For standing waves to form, the wind direction must be near perpendicular to a ridge or range of mountains and the speed must :

  - Decrease with higher within a stable layer above the hill.

  - Increase with higher within an unstable layer above the hill.

  - Decrease with higher within an unstable layer above the hill.

  - Increase with higher within a stable layer above the hill.

- A north/south mountain range, height 10,000 ft is producing marked mountain waves. The greatest potential danger exists for an aircraft flying:

  - On the windward side of the ridge.

  - At FL 350 over and parallel to the ridge.

  - Towards the ridge from the lee side at FL 140.

  - Above a line of clouds parallel to the ridge on the lee side at FL 25.

- Clear air turbulence, in association with a polar from jet stream in the Northern Hemisphere, is more severe:

  - Underneath the jet core.

  - In the centre of the jet core.

  - Looking downstream on the right hand side.

  - Looking downstream on the left hand side.

- Mountain waves can occur:

  - Up to a maximum of 5000 ft above the mountains and 50 nm to 100 nm downwind.

  - Up to mountain height only and 50 nm to 100 nm downwind.

  - Above the mountain and downwind up to a maximum height at the tropopause and 50 nm to 100 nm downwind.

  - In the stratosphere.

- CAT should be reported whenever it is experienced. What should be reported if crew and passengers feel a definite strain against their seat or shoulder straps, food service and walking is difficult and loose objects become dislodged:

  - Light CAT.

  - Extreme  CAT.

  - Severe CAT.

  - Moderate CAT.

- WINDS

- For a constant distance between the isobars the geostrophic wind will be greatest at a latitude of:

  - 45o.

  - 60o.

  - 35o.

  - 20o.

- A cyclone and an anticyclone have the same pressure gradient and the same 2000 ft wind speed. This can only happen when:

  - They are at identical latitudes in opposite hemispheres.

  - They are in the same hemisphere but the cyclone is at a lower latitude.

  - They are in the same hemisphere but the anticyclone is at a lower latitude.

  - The cyclone is due north of the anticyclone in the northern hemisphere.

- During the day the surface wind in western Australia in 270/30 kt. After dusk the wind is most likely to be:

  - 255/40.

  - 260/20.

  - 290/20.

  - 290/40

- On a day in summer with a slack pressure gradient, an airfield on the south coast of England has a surface W/V of 080/06 at dawn. The coast/ line is in an East/West direction. The most probable W/V at noon would be:

  - 360/12

  - 080/18

  - 180/18.

  - 080/06.

- A gust of wind often causes a veer in the surface wind (Northern Hemisphere) because:

  - It strengthens pressure gradient force.

  - It increases coriolis force.

  - It increases centrifugal force.

  - It lifts the air above the friction layer.

- The surface wind direction in the Northern Hemisphere is backed from the 2000 ft wind by an average of 30o over land due to:

  - Friction reducing the wind speed, thus reducing the coriolis force, allowing the pressure gradient force to pull the wind across the isobars towards low pressure.

  - The frictional effect at the surface, causing the surface isobars to curve more than those at 2000 ft.

  - The geostrophic wind only flowing between straight and parallel isobars with an unchanging pressure gradient with time, whereas the surface pressure gradient changes diurnally with surface heating.

  - The effect of the centrifugal force. Which is strongest at the surface due to increased curvature.

- When a Fohn wind occurs:

  - There are unstable conditions on the downwind side of the mountain.

  - Air is cooled mainly at the DALR on the upwind side of the mountain and is heated at the SALR on the downwind side.

  - Windspeed must be at least 15 kt at the surface increasing with height and maintaining direction.

  - Air is cooled mainly at the SALR on the upwind side of the mountain and is heated mainly at the DALR on the downwind side.

- The geostrophic wind blows:

  - Parallel to curved isobars.

  - When pressure values are changing rapidly.

  - In a tropical revolving storm.

  - When the coriolis force and pressure gradient force are equal.

- A Katabatic wind can result in:

  - An increase in temperature.

  - Fog in the valley at night.

  - Thunderstorms in summer.

  - Stratus cloud.

- Surface wind		1500 ft wind

- 250/16			275/25

- The information above shows the winds at the surface and at 1500 ft for mid-day at an airfield in central England. The 2000 ft wind was 290/30.

- Assuming the 2000 ft wind remains the same, the most likely combination of the surface and 1500 ft winds at midnight is:

- Surface wind		1500 ft wind

  - 250/8			260/25

  - 240/10			290/30

  - 250/10			290/30

  - 240/10			275/30

- A strong wind that rises suddenly, last for some minutes and dies comparatively suddenly away is called:

  - A gust.

  - A squall.

  - A gale.

  - A blast.

- The force which causes the air to flow parallel to the curved isobars is called:

  - Pressure force.

  - Centrifugal force.

  - Coriolis force.

  - Gradient force.

- The deflection of the surface wind direction from the geostrophic is, on average:

  - Over the land by night – 35o in cloudy conditions.

  - Over the land by day  – 35o in cloudy conditions.

  - Over the sea – 30o.

  - over the sea - 100 , veered in the southern hemisphere.

- A ravine or valley wind:

  - Is a wind blowing at increased speed along a valley.

  - Is a wind blowing from a hill to a valley below.

  - Is a wind blowing up a hill from a valley.

  - Is a wind blowing at reduced speed along a valley.

- In an anticyclone at 2000 ft, in the northern hemisphere, the wind was 060/20 kt. The likely surface w/v over land is:

  - 070/15 kt.

  - 050/10 kt.

  - 030/10 kt.

  - 030/15 kt.

- UPPER WINDS

- How do you recognize high level jet streams and associated CAT?

  - High pressure centre at high level.

  - Streaks of Cirrus.

  - High level dust.

  - Lenticularis.

- What type of jet stream blows constantly through the northern hemisphere?

  - Arctic.

  - Equatorial.

  - Polar night.

  - Sub tropical.

- In central Europe, where are the greatest wind speeds?

  - Tropopause level.

  - 5500m.

  - Where the air converges.

  - Above the Alps.

- The Arctic Jet core is at:

  - 20,000ft.

  - 30,000ft.

  - 40,000ft.

  - 50,000ft.

- FL 180, Northern Hemisphere with a wind from the left, what can you say about temperature with a heading of 360o?

  - Not possible to tell without a pressure.

  - Increases from South to North.

  - Increases from North to South.

  - Nothing.

- When heading South in the Southern Hemisphere you experience Starboard drift:

  - You are flying towards a lower temperature.

  - You are flying away from a lower temperature.

  - You are flying towards a low pressure.

  - You are flying out of a high.

- With a polar front jet stream ( PFJ), the area with the highest probability of turbulence in the Southern Hemisphere is:

  - In the jet core.

  - Above the jet core in the boundary of the warm and cold air.

  - Looking downstream, on your left hand side.

  - Looking downstream, on your right hand side.

- Contours on a weather chart indicate:

  - Heights of pressure levels.

  - Distance between pressure levels.

  - Thickness between pressure levels.

  - Height of ground.

- If an Isohypse on a surface pressure chart of 500hPa shows a figure of 522, this indicates:

  - Topography of 522m above MSL.

  - Topography of 522m decameters above MSL.

  - Pressure is 522 mb.

  - A low surface pressure.

- The polar front jet stream in summer compared to winter in the Northern Hemisphere moves.

  - North and decreases in strength.

  - North and increases in strength.

  - South and decreases in strength.

  - South and increases in strength.

- A jet stream with a wind speed of 350kts is

  - Impossible.

  - Possible but very rate.

  - Possible in polar areas.

  - Common.

- If you fly at right angles to a jet stream in Europe with a decreasing outside air temperature, you will experience:

  - Increasing headwind.

  - Increasing tailwind.

  - Wind from the left.

  - Wind from the right.

- On a particular day the PFJ runs north to south in the Northern Hemisphere:

  - The temperature gradient runs north to south below the jet core.

  - The temperature gradient runs north to south aboveet core.

  - The polar air is east of the jet above the core.

  - The polar air is below the jet to the east.

- Flying 2500 ft below core of jet, with temperature increasing in the Southern Hemisphere, where does the wind come from?

  - Head.

  - Tail.

  - Left.

  - Right.

- When flying from south to north in the Southern Hemisphere, you cross over the Polar Front Jet. What happens to the temperature?

  - It increases.

  - It decreases.

  - It remains the same.

  - Impossible to determine.

- The core of a jet stream is located.

  - At the level where temperature change with altitude becomes little or nil and the pressure surface is at maximum slope.

  - In the warm air where the pressure surface is horizontal.

  - In the warm air and directly beneath at the surface.

  - In cold air.

- What is the ratio of height to width in a typical jet stream?

  - 1:10.

  - 1:100.

  - 1:1000.

  - 1:10000

- When and where does an Easterly jet stream occur?

  - All year through the Equator.

  - In summer from SE Asia through S. India to Central Africa.

  - In summer from the Middle East through N. Africa and the Mediterranean to S. Spain

  - In winter in Arctic Russia.

- From the pre-flight briefing you know a jet stream is at 3100 ft whilst you are at FL 270. You experience moderate C.A.T. What would be the best course of action?

  - Stay level.

  - Descend.

  - Climb.

  - Reduce speed.

- What is most different about the Equatorial Easterly jet stream?

  - Its height.

  - Its length.

  - Its direction.

  - Its speed.

- Where are easterly and westerly jets found?

  - Northern hemisphere only.

  - Southern hemisphere only.

  - Northern and southern hemisphere.

  - There are no easterly jets.

- Wind at altitude is usually given as ………. In …….

  - True, m/s

  - Magnetic, m/s.

  - True, KT

  - Magnetic, KT

- Under which of the following circumstances is the most severe CAT likely to be experienced?

  - A westerly jet stream at low altitude in the summer.

  - A curved jet stream near a deep trough.

  - A straight jet stream near a low pressure area.

  - A jet stream where there is a large spacing between the isotherms.

- CLOUDS

- The size of raindrops from a cloud is greater if:

  - Air is stable and cloud is layer type.

  - Air is unstable and cloud is heap type.

  - Cloud type is AC or NS.

  - The relative humidity is high.

- Which piece of equipment is used to measure cloud base:

  - Barograph.

  - Hygrometer.

  - Alidade.

  - Barometer.

- In aerodrome reports and forecasts the height of the cloud base refers to:

  - The height above mean sea level.

  - The height above aerodrome elevation.

  - The pressure altitude of the cloud base.

  - The height above the average ground level for the area.

- Cloud types are classified under three basic family headings, these are:

  - Cirriform, Stratiform, Cumuliform.

  - Stratiform, Heap, Cirriform.

  - Cirrus, Cumulus and layer.

  - Cumulus, layer and Heap.

- Low cloud in temperate climates, excluding heap are those existing from:

  - The surface to 6500 ft.

  - 1000 ft to 6500 ft.

  - The surface to 7500 ft.

  - The surface to 7500 metres.

- Lumpy or heaped white cloud is:

  - Altocirrus.

  - Cumulus.

  - Cumulonimbus.

  - Nimbostratus.

- In temperate latitude which statement is correct:

  - Ci only occurs above 15000 ft.

  - Ci only occurs above 16500 ft.

  - Ci only occurs above 25000 ft.

  - Ci only occurs above 30000 ft.

- Typical base heights in temperate latitudes, of altocumulus cloud are in the range of:

  - 6500 ft to 23000 ft.

  - 5000 ft to 15000 ft.

  - 8500 ft to 18000 ft.

  - 1000 ft to 6500 ft.

- Continuous type precipitation is expected from layer clouds because:

  - The vertical extent of the cloud is small.

  - The upcurrents are not very strong.

  - The air is stable.

  - The cloud is large in horizontal extent.

- The expected average height ranges for low, medium and high layer clouds in temperate latitude are:

  - Low:Surface – 10000; Medium 10’000 – 20 000’ high 20 000’ – 45 000’

  - Low:Surface – 6500; Medium 6500’ – 23 000’ high 23 000’ – 45 000’

  - Low:Surface – 8000’; Medium 6500’ – 16 500’ high 16 500’ – 45 000’

  - Low:Surface – 6500; Medium 6500’ – 23 000’ high 16 500’ – 45 000’

- The turbulence associated with cumulus cloud is:

  - Moderate.

  - Slight to nil.

  - Nil.

  - Moderate possible severe.

- When the met observer reports the amount of cloud present at a station, it will be given as:

  - Clear, scattered, broken or overcast.

  - The amount of cloud, in tenths of the sky covered, using the term oktas.

  - The amount of cloud, in quarters of the sky covered, using the term oktas.

  - The amount of cloud, in eighths of the sky covered, using the term oktas.

- On weather charts, the station circle shows the height of the cloud base:

  - Above ground level.

  - Above mean sea level.

  - At a pressure altitude.

  - In metres above surface level.

- Which of the following clouds in never a hazard to aviation:

  - Nimbostratus.

  - Noctilucent cloud.

  - Cummulonimbus cloud.

  - Stratus cloud.

- CLOUDS FORMATION & PRECIPITATION

- Tropical Maritime Air over UK which originally had a surface temperature of 10oC and a lapse rate of 2oC per 1000 ft is subjected to frictional turbulence from the surface up to 3000 ft. Use this information to answer the next two questions:

- After mixing and assuming the air remains dry, the surface temperature is likely to be:

  - 11.5oC.

  - 16oC.

  - 7oC.

  - 10oC.

- The temperature at 5000 ft is likely to be:

  - 0oC.

  - -2oC.

  - +2oC.

  - +3oC.

- If you observe drizzle falling, the cloud above you is most likely to be:

  - AS.

  - CU.

  - ST.

  - NS.

- Turbulence cloud is usually a sheet of Stratus, Strato Cumulus some 2000 ft thick with a flat top because:

  - The air is usually at low temperatures containing little water vapour.

  - Turbulence steepens the lapse rate producing an inversion above the friction layer.

  - Air is not allowed to remain in contact with the surface due to the strong wind thus maintaining cool surface air with warm air above.

  - The lapse rate becomes stable in the friction layer, due to turbulent mixing.

- Clouds formed by convection will always:

  - Be layer clouds.

  - Be CU CB or NS.

  - Have a rising cloud base and may develop into CB as the day progresses.

  - Form only in Polar maritime air.

- With reference to anticyclones affecting the UK, which of the following statements is correct:

  - The pressure gradient is greatest towards the centre of the anticyclone.

  - Anticyclones are more common in winter than they are in summer. This is why radiation fog is much more frequent in the winter.

  - Apart from turbulence cloud, the formation of all other cloud types is unlikely in anticyclonic conditions.

  - Warm anticyclones are those which are caused by the extreme air density associated with warmer weather.

- The type of cloud from which continuous moderate or heavy rain is likely to fall is:

  - Large cumulus.

  - Altostratus.

  - Nimbostratus.

  - Cumulonimbus.

- The movement of cool moist air over a warmer surface is likely to cause:

  - Cumulus or cumulonimbus cloud.

  - Advection fog.

  - Nimbostratus cloud.

  - Alto cumulus lenticular cloud

- Over the British Isles, a polar maritime returning airmass would have:

  - Stability.

  - Instability.

  - Potential instability.

  - Neutral stability.

- Intensity of precipitation is described as either:

  - Intermittent, continuous or showery.

  - Drizzle, rain or snow.

  - Slight, moderate or heavy.

  - Intermittent, moderate or heavy.

- The term “ Shower” implies that:

  - Precipitation is in the form of rain and is continuous.

  - Precipitation is from cumulonimbus cloud and lasts for short periods.

  - Precipitation is intermittent and is from strato form cloud.

  - Precipitation is continuous for long periods from cumuloform cloud.

- Precipitation in the form of snow will not reach the surface unless the surface temperature is:

  - Less than +4oC.

  - Less than 0oC.

  - Less than 45oF.

  - Less than 30oF.

- The type of precipitation in which visibility is likely to be most reduced:

  - Drizzle.

  - Snow.

  - Light rain.

  - Sleet.

- The type of precipitation usually associated with shallow stratocumulus is:

  - Mainly water droplets which can be supercooled if the temperature is low enough.

  - Ice crystals.

  - Supercooled water droplets only.

  - Large water droplets due to the strong up-currents associated with this type of cloud.

- If there are small cumulus in the morning in summer, it is reasonable to forecast later in the day:

  - Clear skies.

  - St. and drizzle.

  - CB Cloud.

  - Haze.

- THUNDERSTORMS

- The conditions which must exist to allow thunderstorms to develop are:

  - A trigger action, a plentiful supply of moisture and a very stable atmosphere.

  - A steep lapse rate, a stable atmosphere through a large vertical extent and a plentiful supply of moisture.

  - A plentiful supply of moisture and a steep lapse rate through a large vertical extent and trigger action.

  - A steep lapse rate through a large vertical extent, a low relative humidity and a trigger action.

- When moist air moves across France in the ………. TS activity is common in southern UK in the ……….

- Complete the above statement correctly using one of the following:

  - Winter/morning.

  - Summer/late afternoon or evening.

  - Winter/late afternoon or evening.

  - Summer/morning.

- Hazards of the mature stage of a TS cell include lighting, turbulence and:

  - Microburst, windshear and anvil.

  - Icing, microburst and windshear.

  - Icing, drizzle and microburst.

  - Windshear, hail and fog.

- On a significant weather chart the thunderstorm symbol signifies:

  - Moderate turbulence and moderate icing.

  - Severe turbulence and severe icing.

  - Moderate turbulence and severe icing.

  - Moderate/severe turbulence and/or moderate severe icing.

- Thunderstorms require a trigger action to release the conditional instability. Which of the following would be the least suitable as a trigger:

  - Convergence in temperate latitudes.

  - Convergence in tropical latitudes.

  - Subsidence in tropical latitudes.

  - Convection in polar latitudes.

- During the ……….. stage of a thunderstorm cell, the cloud contains …………

- Complete the above statement correctly using one of the following:

  - Building/up currents and down currents.

  - Mature/up currents and down currents.

  - Dissipating/up currents and down currents.

  - Building/down currents only.

- The following is unlikely to be a hazard below a thunderstorm:

  - Severe turbulence.

  - Severe icing.

  - Windshear.

  - Large variations in pressure setting values.

- Thunderstorms are likely if:

  - Air is unstable, there is sufficient water vapour and there is trigger action.

  - Air is completely stable, there is sufficient water vapour and there is lifting orographically.

  - There is a warm front.

  - There is a col in winter.

- Hail grows by:

  - Freezing as it leaves the cloud.

  - By up and down progress in CU cloud.

  - By collision with supercooled water drops.

  - By collision with ice crystals.

- How long approximately does a Cumulonimbus cell take to complete the full cycle from the Cumulus ( building) to dissipating stage:

  - 2-3 hours.

  - 1-2 hours.

  - 4-5 hours.

  - About 1 hour.

- When approaching at flight level 300 a cumulonimbus cloud with an anvil top, pilots should air to avoid the cloud by  --------- nms horizontally if avoiding visually, or by ------ nms horizontally if using cloud avoidance radar. Select the appropriate respective ranges from those given below:

  - 10		20

  - 15		10

  - 10		15

  - 5		10.

- A microburst usually lasts for _________ and is about ________ across.

  - 20 minutes		20 nm.

  - 5 minutes		5 km.

  - 30 minutes		10 nm.

  - 45 minutes		5 nm.

- Thunderstorms caused by __________ are most common in the summer and by _______ in the ________

  - Lapse rate		air masses		late spring.

  - Air masses		frontal activity  winter

  - Cold fronts		air masses		autumn

  - Air mass		frontal activity   summer.

- When flying through an active CB cloud , lightning strikes are most likely:

  - Above 5000’ and underneath the anvil.

  - In the clear air below the cloud in rain.

  - In the temperature band between +10oC and -10oC.

  - At or about 10 000 ft AMSL.

- Regarding thunderstorms, the most accurate statement amongst the following is:

  - There will always be windshear under the cloud.

  - The average movement is in accord with the wind at 10 000 ft.

  - If the cloud base has a temperature below 0oC then freezing rain will occur.

  - The number of lightning flashes is directly proportional to the degree of turbulence.

- VISIBILITY

- Which of the conditions given below will lead to the formation of radiation fog?

- Wind speed	Cloud Cover		Temperature		Dew Point.

  - 7 kt			8/8 St.			12oC			11oC.

  - 15 kt		NIL			15oC.			14oC.

  - 3 kt			1/8 Ci			8oC			7oC

  - 2 kt			NIL			- 2oC			- 3oC

- When ________ moist air passes over a surface which is _______ than the dew point of the air, ________ fog can form. This occurs over __________

- Examine the statement above; the line which contains the correct words in the correct order to complete the statement is:

  - Cool	warmer	radiation	the sea.

  - Warm	cooler		radiation	the land only.

  - Cool	warmer	frontal		land.

  - Warm	cooler		advection	land and sea.

- Radiation fog is most likely at an inland airfield in the UK with a relative humidity of 80% in the ________ with ________ and a wind of ________

  - Autumn		clear skies		2-8 knots.

  - Spring		6/8 ST & SC		2-10 knots.

  - Winter		clear skies		15/20 knots.

  - Summer		clear skies		no wind.

- Advection fog is formed when __________ air moves over a _______ surface and is ________ its dewpoint:

  - Humid	cold		kept above.

  - Warm moist	cooler		cooled below.

  - Dry			frozen		well below.

  - Warm moist	warmer	kept above.

- On a night when radiation fog is forming over most of southern England, the aerodromes likely to be first to experience the fog will be those situated:

  - Near the coast with a light onshore wind and clear skies.

  - At the bottom of the hill with a light katabatic wind blowing.

  - Near the coast with a land breeze and cloudy skies.

  - At the top of a hill with clear skies and no wind.

- Radiation fog is most likely:

  - With a wind speed up to 15 kt, a clear sky and a high relative humidity.

  - With a wind of 2-8 kt, a high density and the summer season.

  - In an anticyclone in winter.

  - On a hill in autumn.

- If a station equipped with IRVR equipment reports RVR 1000, this means :

  - RVR at touchdown is 1000 metres.

  - RVR at touchdown is 1000 metres and at ‘mid point’ and ‘stop end’ the RVR is 800 metres or more.

  - RVR at touchdown is 1000 metres and ‘mid point’ and ‘stop end’ equipment is unserviceable.

  - RVR all along the runway is 1000 metres or more.

- Changes of RVR are reported for increments of:

  - 25 m up to 250 m.

  - 25 m up to 200 m.

  - 50 m between 300 m and 800 m.

  - 50 m between 500 m and 800 m.

- Frontal fog is most likely to:

  - Form ahead of a vigorous fast moving cold front

  - Form ahead of a warm front.

  - Form on a vigorous cold front and last for many hours.

  - Form to the rear of a warm front but only last for 1 to 2 hours.

- Fog may be defined as:

  - A reduction of visibility to less than 1000 metres due to the presence of water vapour in the atmosphere.

  - A reduction of visibility to less than 1000 metres due to the presence of water droplets  in suspension in the  atmosphere.

  - A reduction of visibility to less than 1500 metres due to the presence of water droplets  in suspension in the atmosphere.

  - A reduction of visibility to less than 1000 ft. due to the presence of water vapour in the suspension in the atmosphere.

- Several types of pressure distribution may be associated  with radiation fog but all have one feature in common which is:

  - Closely spaced isobars.

  - A tight pressure gradient.

  - A slack pressure gradient.

  - A rapidly falling pressure.

- ………… forms when moist air ……… over a surface which is …………. Than the dew point of the air. Fill in the missing 3 words from the list given below:

  - Radiation fog, passes, warmer.

  - Advection fog, settles, cooler.

  - Advection fog, passes, cooler.

  - Radiation fog, settles, warmer.

- Advection fog:

  - Only occurs at night and early morning.

  - Is most likely with Polar Maritime air.

  - Will only clear by insolation.

  - Can sometimes last for 24 hours or more in winter.

- In circumstances where there is a clear sky, calm wind and a high relative humidity in Autumn:

  - Radiation fog is likely over night.

  - Advection for will form.

  - Radiation fog is likely at sunrise after previous mist.

  - Hill fog can be expected.

- At a station equipped with IRVR, reports are given:

  - Every ½ hour.

  - When the normal visibility is 1500 m or less.

  - When there is mist.

  - When there is haze.

- ICING

- At temperatures of between 0oC and -10oC clouds will consist of:

  - Entirely water droplets.

  - Entirely ice crystals.

  - Mostly water vapour.

  - Mostly supercooled water droplets and a few ice crystals.

- Turbulent clouds are most serious from the icing standpoint because:

  - Strong vertical currents mean that a predominance of large supercooled water droplets will be present.

  - Strong vertical currents mean that a predominance of small supercooled water droplets will be present.

  - Turbulent clouds produce hail which sticks to the aircraft.

  - Turbulent clouds indicate a low freezing level.

- Hoar frost forms on an aircraft when:

  - The aircraft suddenly enters a cloud at below freezing temperature.

  - The aircraft in sub zero clear air suddenly enters a colder region.

  - The aircraft in sub zero clear air suddenly enters a warmer moist region.

  - The aircraft suddenly enters a cloud which is at a higher temperature than the surrounding air.

- Most cases of serious piston engine icing occur in cloud, fog or precipitation with a temperature range between:

  - -10oC		to		+25oC

  - -18oC		to		+5oC

  - -10oC		to		+0oC

  - -20oC		to		+15oC

- Stratus cloud of limited depth at a temperature of -50C will most likely give:

  - Moderate to heavy rime ice.

  - Moderate to heavy glaze ice.

  - Light to moderate rime ice.

  - Light to moderate glaze ice.

- Clear ice forms as a result of:

  - Large supercooled water droplets spreading as they freeze.

  - Ice pellets splattering on the aircraft.

  - Small supercooled water droplets splashing over the aircraft.

  - Water vapour freezing to the aircraft.

- Orographic uplift in stable conditions gives a strong vertical component to air movement thus supporting larger supercooled droplets in orographically formed cloud. Consideration should also be given to the fact that in this cloud:

  - The 0oC isotherm will be higher.

  - The 0oC isotherm will be lower.

  - The lapse rate will be isothermal.

  - An inversion can be anti-cyclonic.

- Which of the following conditions is most favourable for the formation of carburetor icing if the aircraft is descending with glide power set:

- Relative Humidity		Ambient Temperature

  - 25%			+25oC

  - 40%			+20oC

  - 50%			-10oC

  - 30%			-5oC

- Flying in large CU at a temperature of -20oC, the amount of each cloud droplet that will freeze on impact with the aircraft will be:

  - All the droplet.

  - ½ of the droplet.

  - ¼ of the droplet.

  - 20% of the droplet.

- Carburettor icing is unlikely:

  - In cloud.

  - At temperatures between -10oC and -30oC.

  - In clear air.

  - When the RH is 40%.

- Flying 50 nm ahead of a warm front out of cloud at 1000 ft in winter, with an ambient temperature of -8oC, there is a strong risk of:

  - Hoar frost.

  - Rime icing and carburetor icing.

  - Structure damage caused by hail.

  - Clear ice in the form of rain ice.

- In AS cloud at FL 170 and a temperature of -20oC the airframe icing most likely to be experienced is:

  - Moderate clear icing.

  - Light rime icing.

  - Hoar frost.

  - Severe clear icing.

- Mixed ( rime and clear) icing is most likely to be encountered:

  - In nimbostratus at a temperature of  -10oC

  - In stratocumulus cloud  at a temperature of  -20oC

  - In fair weather cumulus at a temperature of -15oC.

  - In towering  cumulus at a temperature of -10oC.

- When considering icing in cloud over high ground compared with icing in other clouds, the effect of orographic lifting is to:

  - Cause the height of the freezing level to fall and increases the intensity of the icing.

  - Cause the height of the freezing level to rise and increases the severity of the icing.

  - Cause the free water content of the cloud to increase and the freezing level to rise so reducing the  icing risk.

  - Increase the temperature inside the cloud due to the release of extra latent heat so reducing the icing risk.

- Kinetic heating will:

  - Increase the risk of icing if it raises the airframe temperature to just below 0oC.

  - Increase the risk of icing if it raises the airframe temperature to just above 0oC.

  - Always increase the risk of airframe icing.

  - Always decrease the risk of airframe icing.

- DOCUMENTATION

- Frankfurt Maine/EDDF : The lowest cloud present at 201720 was:

  - 1 to 2 oktas at 4, 500 feet.

  - 1 to 2 oktas at 4, 50 feet.

  - 5 to 7 oktas at 2,500 feet.

  - 3 to 4 oktas at 2,50 feet.

- Milan/LIML : The lowest visibility to be EXPECTED AT 202100 IS:

  - More than 10 km.

  - 3000 m.

  - 5000 m.

  - 900 m.

- Geneva/LSGG : The lowest visibility observed on the 20th is:

  - 5000 metres to the north east.

  - 5000 metres to the north .

  - 4000 metres to the north east.

  - 7000 metres to the south .

- At which of these stations and at what time is the highest relative humidity to be found:

  - EDDF at 1650.

  - LIRA at 1720.

  - LIML at 1720.

  - LSGG at 1650.

- Milan/LIML : What is the visibility forecast to be at LIML at 201730:

  - 3000m.

  - 8000m.

  - 5000m.

  - 10km or more.

- WEATHER CHARTS

- Using the attached significant weather chart ( Appendix A) answer question 1 to 4 on the route from madrid to Larnaca following the marked route.

- The highest tropopause height en-route would be at:

  - Overhead 20E

  - Overhead Madrid.

  - Lornaca

  - 10oE

- The CAT expected at 5oE en-route would be:

  - Moderate between FL 230 and FL 460.

  - Moderate from below FL 100 to FL 160.

  - Moderate from FL 260 to FL 370.

  - Moderate between FL 290 to FL 440.

- If this route was flown at FL 290 moderate to severe turbulence and icing could be expected at:

  - 5oE.

  - 10oE.

  - Overhead Larnaca.

  - 25oE.

- Using the attached upper wind and temperature chart ( Appendix B) for flying the route from Madrid to Larnaca at FL 300 answer Questions 4 to 6.

- The mean wind velocity and temperature between Madrid and 30oE would be:

  - 295/70	-40.

  - 290/80	-45.

  - 270/75	 42.

  - 260/70	 38.

- The ISA deviation overhead Madrid is:

  - ISA -4

  - ISA +4

  - ISA +5

  - ISA +3

- The highest ground speed would be achieved at:

  - 25oE.

  - 15oE.

  - 30oE.

  - 20oE.

- Using the attached F 214 UK low level wind and temperature chart ( Appendix C) answer Questions 7 to 9.

- The mean wind and temperature between Livepool ( EGGP) and Jersey ( EGJJ) at 14000 ft would be:

  - 250/50 	-8oC.

  - 280/35 	-5oC.

  - 275/52 	-17oC.

  - 255/60 	-23o.

- At position 57o 30’N 05o 00’W between 2000 ft and 5000 ft the ELR in existence would lead to a state of:

  - Neutral equilibrium.

  - Absolute stability.

  - Conditional instability.

  - Absolute instability.

- The ISA deviation at 55o N 00o E at 18000 ft would be:

  - ISA – 3oC.

  - ISA + 0oC.

  - ISA + 12oC.

  - ISA + 3oC

- Using the attached F215 UK low level forecast ( Appendix D) answers 10 to 15.

- The type of fog forecast in Zone 4 is most likely to be:

  - Radiation fog.

  - Frontal fog.

  - Advection fog.

  - Arctic smoke.

- At 0900 Z the occlusion velocity is:

  - 120o/20kt.

  - 080o/35kt.

  - Less than 5 kt.

  - 090o/30 kt.

- In Zone 3 the reported cloud is:

  - Strato cumulus.

  - Stratus and strato cumulus.

  - Alto cumulus.

  - Stratus, strato cumulus and alto cumulus.

- By 1200 Z the lowest visibility will be found in:

  - Zone 4.

  - Zone 2 and 3.

  - Zone 5.

  - Zone 2, 3 and 4.

- The highest cloud tops expected at Kidlington between 0600 Z and 1200 Z are:

  - FL 250

  - 14000 ft AGL.

  - 8000 ft AMSL.

  - 25000 AMSL.

- The surface wind direction expected at London Heathrow at 1800 Z is:

  - 060oT

  - 290oT

  - 180oT

  - 240oT

- From the TAF’s for MKPP and KBOS given below answer 16 to 20.

- MKPP		270606 10017 Kt 3000 HZ SCT024 PROB30 TEMPO  0812 2000

- +SHRA BKN010CB BECMG 1215 VRB05KT CAVOK BECMG 0103 10010KT 5000 SCT015

- KBOS	271212 VRB05 CAVOK BECMG 1819 06012KT BECMG 0204 05025G35KT 5000 OVCO030 PROB40 1012 2800 SN.

- The visibility at MKPP at 1600 Z is expected to be:

  - 2000 m.

  - 3000 m.

  - 5000 m.

  - 10 km or more.

- The lowest cloud base forecast for MKPP at 1100 is:

  - 2400 ft AMSL

  - Above 5000 ft AMSL.

  - 2400 ft AGL.

  - 1000 ft AGL.

- The highest surface wind speed at MKPP is expected between:

  - 0600 Z and 0600 Z.

  - 0600 Z and 1200 Z.

  - 1200 Z and 1500 Z.

  - 0800 Z and 1200 Z.

- At KBOS at 1100 Z which of the following weather conditions are expected:

  - Visibility of 28 km.

  - 8/8 cloud at 2800 ft.

  - 40% chance or moderate snow.

  - Surface wind of 05025 KT.

- At KBOS at 0600 Z the visibility is expected to be:

  - 5000 m.

  - More than 10 K m.

  - 2800 m.

  - 1012 m.

- AIR MASSES

- An air mass that has traveled over an ocean is known as:

  - Continental air and has a high humidity.

  - Continental air and has a low humidity.

  - Maritime  air and has a high humidity.

  - Maritime  air and has a low humidity.

- Characteristic weather associated with a Pm airmass transitting the British Isles in Summer would include:

  - Widespread Cu and Cb activity overland during the day.

  - Clear quiet settled weather overland by day with good visibility.

  - Warm moist conditions with some Sc or Cu and moderate to poor visibility.

  - Extensive low stratus cloud giving drizzle to light rain overland by day.

- If air in transit is heated from below it tends to become more:

  - Stable.

  - Neutrally stable.

  - Unstable.

  - None of these.

- The weather associated with polar maritime air is:

  - Overcast, moderate drizzle.

  - Overcast moderate intermittent rain.

  - Broken cloud, light moderate or heavy rain.

  - Broken cloud, moderate continuous rain.

- Polar maritime air is …………….. and can bring …………. In the UK in winter but …………. In summer.

- Complete the above sentence correctly using one of the following:

  - Very unstable/heavy snow showers/does not arrive.

  - Cold and stable/advection fog/rain showers.

  - Unstable/intermittent or continuous snow/cool dry weather

  - Unstable/heavy showers/light rain showers.

- Tropical continental air normally brings to the UK:

  - Hot dry cloudless weather with a thick haze.

  - Warm weather with broken Cu and showers on coasts, visibility very good except in showers.

  - Warm dry cloudless weather with very good visibility.

  - Hot dry cloudless weather on coasts but Cu building up inland with rain showers, visibility good except in showers.

- Referring to the area of the North Atlantic, the mean position of the polar front in January is:

  - From Florida to south west England.

  - From Newfoundland to the north of Scotland.

  - From Florida to the north of Scotland.

  - From Newfoundland to south west England.

- When air from an air mass moves to a lower latitude, it can be expected that:

  - Surface layer air will become warmer, the RH will rise and the air will become unstable.

  - Surface layer air will become colder, the RH will rise and the air will become more stable.

  - Surface layer air will become warmer, the RH will fall and the air will become unstable.

  - Surface layer air will become warmer, the RH will fall and the air will become more stable.

- In the N, hemisphere when flying in the troposphere above the surface friction layer in the polar maritime air mass behind the cold front of a fully developed frontal depression:

  - The wind will tend to veer in direction and increase in speed with progressive increase of altitude.

  - The wind will tend to veer in direction and increase of altitude but the speed may remain constant in the lower layers of the atmosphere.

  - The wind speed will reduce progressively with increase of altitude until at about 10000 feet above mean sea level where it will then tend to increase in speed from another direction.

  - The wind will tend to back in direction and increase in speed with progressive increase of altitude.

- The average surface level winds at A3, B3, and C3 in Appendix ‘A’ are respectively:

  - Easterly, westerly, southwesterly,

  - Westerly, westerly, southwesterly.

  - Southwesterly, westerly, northwesterly.

  - Southwesterly, westerly, northerly.

- The average upper  winds at A1, B1, and C1 in Appendix ‘A’ are respectively:

  - Easterly, westerly, northwesterly,

  - Northwesterly, westerly, southwesterly.

  - Southwesterly, westerly, northwesterly.

  - Southwesterly, westerly, northerly.

- It can be expected that the depth of the Friction Layer over the UK will be:

  - Greater in Polar Maritime air due to the instability and moderate wind.

  - Greater in Tropical Maritime  air due to the warm temperature.

  - Greater in Polar Continental air due to the very low temperatures.

  - Greater in Tropical Continental  air due to the relatively high  temperature in winter.

- The airmasses involved in the development of a Polar Front Depression are:

  - Polar Maritime and Polar Continental.

  - Tropical  Maritime and Polar Continental.

  - Tropical Continental  and Polar Maritime.

  - Polar Maritime and Tropical Continental

- When a cold front passes a Station in the British Isles:

  - The wind veers and the dewpoint falls.

  - The wind backs and the dewpoint falls.

  - The wind veers and the dewpoint rises.

  - The wind backs and the dewpoint rises.

- Refer to Appendix B

- The airmasses indicated in the diagrams by the hand are respectively

  - Arctic, Tropical Continental, Polar Maritime, Arctic Martime.

  - Polar Continental, Tropical Maritime, Tropical Continental, Arctic.

  - Polar Martime, tropical Martime, Polar continental, Arctic.

  - Polar Continental, Polar Maritime, Tropical Maritime, Arctic.

- OCCLUSIONS

- Refer to Appendix A for Question 1-3:

- The cloud in grind square M11 is most likely to be:

  - Cirrus.

  - Nimbostratus.

  - Altocumulus.

  - Stratus fractus.

- Precipitation will reach the ground mainly in the area:

  - L14 – R 14.

  - Q 14 - S 14

  - 014 – T 14.

  - J14 – O14.

- In grid square M6 the worst cloud conditions for flying could be:

  - Altrocumulus.

  - Cumulo nimbus embedded in nimbostratus.

  - Cumulonimbus.

  - Nimbostratus.

- Which of the conditions below would lead to the worst icing condition?

- Size of Drop	Ambient Temp.

  - 2 mm		-30oC

  - 1 mm		-1oC

  - 5 mm		-4oC

  - 3 mm		-12oC

- Refer to Appendix ‘B’. In a warm occlusion flying at 20000’ Where will the most turbulence be found?

  - A

  - B

  - C

  - A and C

- Refer to Appendix “C”. Which area will get the most rain at the surface?

  - A

  - B

  - C

  - D

- Refer to Appendix “D”. What type of cloud will be found at X?

  - CS.

  - NS.

  - SC.

  - CB.

- Refer to Appendix ‘D’. What type of cloud is most likely at Z?

  - CU.

  - CB.

  - AS.

  - NS.

- After passage of an occluded front in the Northern Hemisphere:

- Wind	Temperature		Precipitation

  - Backs	stops falling		continues

  - Veers	drops rapidly		stops abruptly.

  - Veers	drops or rises		begins to dry up.

  - Backs	rises quickly		increases in strength.

- With a cold occlusion:

  - The air ahead of the warm front is colder than the air behind the cold front.

  - The warm sector remains on the surface.

  - The cloud type is predominately layer with a wide precipitation band.

  - There is a risk of CB embedded in NS.

- Refer to Appendix “A” for question 11.

- The front at P14 is:

  - Cold.

  - Warm.

  - Warm at an occlusion.

  - Cold at an occlusion.

- Refer to Appendix “B” for questions 12-14

- The relative temperatures at  A, B, and C could be respectively:

  - +6		+8		+10

  - +6		+10		+8

  - +8		+10		+8

  - +10		+6		+8

- Precipitation at the surface underneath B is likely to Be :

  - Light drizzle,

  - Continuous moderate.

  - Showers, heavy with the possibility of hail.

  - Nil.

- Flight conditions at B are likely to be:

  - Smooth and clear.

  - Layer clouds with only light turbulence.

  - Some turbulence in NS with the possibility of embedded. CB giving moderate/severe turbulence.

  - Flight in CU, CB with some turbulence.

- When flying from west to east through a cold occlusion ( below the warm air) over the north Atlantic you would expect the wind to …………… and the temperature to ………..

  - Veer/decrease.

  - back/increase.

  - Back /decrease.

  - Veer/increase.

- A warm occlusion occurs when:

  - Warm air is forcing cool air over cold air.

  - Cold air is forcing cool air over warm air.

  - Cool air is forcing warm air over cold air.

  - Cool air is forcing cold air aloft.

- OTHER DEPRESSIONS

- A thermal depression is likely to form:

  - Over the Iberian peninsular during the summer.

  - In the lee of the Alps over Northern Italy in winter.

  - In association with a marked trough of low pressure over the USA.

  - On the trailing edge of a warm sector mid latitude depression.

- Tropical revolving storms usually:

  - Form close to one side of the equator and while moving slowly in a westerly direction cross over to the other hemisphere.

  - Move in a westerly direction before recurving towards the equator.

  - Move in an easterly direction before recurving towards the nearest pole.

  - Do not form within 5o of the equator.

- With reference to tropical revolving storms, which of the following statement is correct:

  - Typhoons are found in the South China sea January.

  - Cyclones, occure in the Bay of Bengal in winter.

  - Hurricanes, in the South Atlantic sometimes affect the east coast of Brazil.

  - Hurricanes affect the south east of the USA in late summer.

- Which of the following statements accurately describes the “West African Tornado”.

  - The West African Tornado is similar to the North American and European tornadoes.

  - It is a line of thunderstorms producing a line squall aligned roughly north/south.

  - It is another name for the cyclones that affect the West African coast in summer.

  - It is the name given to a line of thunderstorms that lie along the ITCZ but some 200 miles to the south.

- Extensive cloud and precipitation is often associated with a non-frontal thermal depression because of:

  - Surface divergence and upper level convergence causing widespread descent of air in the depression.

  - Surface convergence and upper level divergence causing widespread descent of air in the depression.

  - Surface convergence and upper level divergence causing widespread ascent of air in the depression.

  - Surface divergence and upper level convergence causing widespread ascent of air in the depression.

- In comparison with a primary depression a secondary depression is:

  - Always more active.

  - Sometimes more active.

  - Never more active.

  - Unlikely to produce gale force winds.

- A secondary Depression would form in association with:

  - A Polar Depression.

  - A Col.

  - A Summer Thermal Depression over the Mediterranean or Caspian Sea.

  - A Polar Front Low.

- Tropical Revolving Storms:

  - Are always given a male first name beginning with “A” for the first of the season and thereafter named in alphabetical order of occurrence.

  - Have internal wind speeds of 10-20 knots rotating cyclonically round a subsiding clear air core known as the eye.

  - Usually have the most severe weather in the quadrant to the right of the track in a Hurricane.

  - Regenerate after crossing the coast from sea to land.

- Which of the following are Thermal Depressions:

  - Tropical revolving storms, polar air depressions, tornadoes.

  - The equatorial trough, monsoon lows, some depressions over the central and eastern Mediterranean sea in summer.

  - The equatorial trough, polar air depressions, monsoon lows, orographic lows.

  - The lows forming over flat land in summer, polar air depressions, tropical revolving storms, some of the lows which form over inland seas in winter.

- A Secondary low pressure system rotates around a Primary low:

  - Cyclonically.

  - Ant cyclonically.

  - Into the primary.

  - At a constant distance.

- Flying conditions in a Secondary low pressure system are:

  - Always more severe than in a Primary low.

  - Sometimes more severe than in a Primary low.

  - Less severe than in a Primary low.

  - Relatively calm.

- Tropical Revolving Storms:

  - Do not occur in the South Atlantic.

  - Generally move from east to west before turning towards the equator.

  - Intensify after crossing coasts.

  - Occur principally in spring and early summer.

- GLOBAL CLIMATOLOGY

- The Savannah climatic zone is:

  - Approximately 20o – 30o of latitude and provides very dry desert conditions throughout the year.

  - Approximately 10o – 20o of latitude and provides  dry trade wind  conditions in winter and a wet summer season.

  - Approximately 10o – 20o of latitude and provides a wet winter season and a dry hot summer.

  - Approximately 10o – 30o of latitude and has a period of long rains in spring and autumn, but is never dry.

- The warm temperature climate zone is:

  - Approximately 20o – 35o of latitude and covers the high pressure desert regions of the world.

  - Approximately 35o – 40o of latitude and is under the influence of polar front depressions throughout the year.

  - Approximately 35o – 40o of latitude and provides a warm dry summer with  a cool wet winter.

  - Approximately 35o – 40o of latitude and provides  a wet summer season and a dry cold winter.

- Northern hemisphere summers tend to be:

  - Warmer than the southern hemisphere and winters are warmer too.

  - Colder than the southern hemisphere due to the smaller amount of solar radiation.

  - Colder than the southern hemisphere because of the large and masses.

  - Warmer than the southern hemisphere and the winters are colder.

- The effect of mountain barriers on temperature is exemplified by the following:

  - European temperatures are low in winter because there is no barrier to prevent cold Atlantic air crossing the area.

  - The Rocky mountains of North America prevent cold Pacific air reaching inland, so summer temperatures to the east of the mountains are high.

  - The Himalayas prevent warm dry air from Russia reaching India and Pakistan.

  - The Ural mountains of West Russia prevent most of the cold Siberian air reaching Europe in summer.

- The Tropopause is:

  - More or less constant for any latitude.

  - Is a boundary layer between the troposphere and the stratosphere.

  - Normally the upper limit of weather.

  - The upper limit for jet streams and mountain waves.

- Statistical pressure values tend to be:

  - On average parallel to the lines of latitude.

  - On average parallel to the lines of latitude in the southern hemisphere and much more variable in the northern hemisphere.

  - Much lower in winter in the northern hemisphere than in the southern hemisphere.

  - Higher over the oceans in winter.

- The heat equator is:

  - Another name for the geographic equator.

  - Coincident with the equatorial trough and ITCZ.

  - A line over the land joining places where the summer temperatures are highest.

  - A line over the land joining places where the winter temperatures are highest.

- The average temperature around the equatorial regions:

  - Is always above + 40oC.

  - Is higher over the sea areas.

  - Varies on average from winter to summer by only some 50C.

  - Has a very high range of temperatures throughout the year.

- Trade winds:

  - Blow towards the sub tropical anticyclones.

  - Are caused by lifting over the heat equator and the subsequent air movements from the sub tropical anticyclones.

  - Only blow in the winter months.

  - Blow from the equatorial low pressure systems throughout the year.

- Trade winds in the southern hemisphere are:

  - Southeasterly.

  - Southeast at first becoming southwest.

  - In opposition to the monsoons.

  - Usually from the northest.

- The Hadley cell is the name given to:

  - The CU cells which continue to form a CB.

  - An initial bubble of air which is lifted by convection.

  - The centre portion of a jet stream.

  - A Cell formed by lifted air over the heat equator descending to the sub tropical highs.

- The large change in the direction of trade winds is caused by:

  - Local pressure differences.

  - An excess of air at height in association with the Hadley cells.

  - The change in geostrophic force when crossing the geographic equator.

  - The cyclostrophic  force in the equatorial regions.

- Monsoons are seasonal winds which:

  - Develop due to the high pressure over continents in winter and the subsequent low pressure which develops over the same areas in summer.

  - Are never in combination with trade winds.

  - Blow only in the southeast Asia region.

  - Are from the southeasterly direction over the Indian sub continent in summer.

- The outflow from the Siberian High:

  - Is northwesterly over Japan, northerly and northeasterly over China and northerly over the whole of India.

  - Is the source of Polar Maritime air.

  - Is northwesterly over Japan, northeasterly over southeast Asia and easterly over Europe.

  - Is evident throughout the year.

- The upper winds tend to be westerly outside the tropics because:

  - The rotation of the earth is west to east.

  - The thermal winds are westerly on average.

  - Surface winds are nearly always westerly.

  - Jet streams are usually westerly.

- Jet stream main locations are:

  - In the warm air some 400 nm ahead of a warm or cold front and near the subtropical highs.

  - In the warm air some 400 nm ahead of a warm front and some 200 nm behind a cold front and near the sub tropical highs.

  - Only in association with the polar front.

  - In association with the polar front and with mountain waves.

- Jet streams:

  - Only occur in the troposphere.

  - Have a speed in excess of 80 kt.

  - Are located above the tropopause.

  - Are caused by a large difference in mean temperature in the horizontal.

- Near the equator upper winds tend to be:

  - Easterly.

  - Westerly.

  - At speeds greater than 60 kt.

  - Calm.

- The Polar Front is:

  - The boundary surface between polar continental and tropical continental air.

  - Near the poles.

  - Only apparent over the Atlantic ocean.

  - The region where warm sector depressions develop.

- The ITCZ is:

  - The region between the two trade wind systems centred on the heat equator.

  - The boundary region between the two monsoons.

  - The boundary between polar air and equatorial air.

  - A region of calm winds and layer type clouds with much haze.

- Tropical Revolving Storms are :

  - A summer weather feature.

  - Easily predictable.

  - Can be very active well inland.

  - Can travel as speeds of 100 kt.

- The areas of greatest rainfall are:

  - Those where there is much polar front depression activity.

  - In the equatorial regions.

  - In the polar regions.

  - In central North America in summer due to the large convective cloud formations.

- Thunderstorms most frequently occur:

  - In association with the ITCZ over central Africa.

  - Over the east Indies area (Java) due to the intense surface heating.

  - In regions affected by cold fronts.

  - In association with tropical revolving storms.

- Dust storms and haze are most common:

  - In association with the sub tropical anticyclones over land.

  - With the Haboobs in winter.

  - In unstable air with low pressure.

  - In temperate latitudes.

- The  most notorious advection fogs occur:

  - Over southwest UK.

  - Over the sea in the region of Newfoundland and the Kamchatka peninsula.

  - Over Europe with high pressure to the north.

  - Over central North America in autumn and winter.

- The cloud to be expected along the front at A3 in Appendix A is:

  - CU CB.

  - ST SC.

  - ST NS.

  - AS NS.

- The cloud to be expected at B2 between the fronts in Appendix A is:

  - AS.

  - ST SC.

  - NS.

  - NIL.

- The cloud to be expected at C2 along the front in Appendix A is:

  - CU CB

  - AS NS.

  - ST SC.

  - AC.

- The cloud to be expected along the front at A2 in Appendix A is:

  - CI.

  - AS NS.

  - ST.

  - CU

- The average surface level winds at A3, B3, and C3 in Appendix A are respectively:

  - Easterly, westerly, southwesterly.

  - Westerly, westerly, southwesterly.

  - Southwesterly, westerly, northwesterly.

  - Southwesterly, westerly, northerly.

- The average upper winds at A1, B1, and C1 in Appendix A are respectively:

  - Easterly, westerly, northwesterly.

  - Northwesterly, westerly, southwesterly.

  - Southwesterly, westerly, northwesterly.

  - Southwesterly, westerly, northerly.

- The names of the air masses indicated A, B, C and D at Appendix B are respectively:

  - Polar Maritime, Polar Continental, Tropical Maritime, Tropical Continental.

  - Returning Polar Maritime, Arctic, Tropical Continental, Tropical Maritime.

  - Polar Maritime, Arctic, Tropical Continental, Tropical Maritime.

  - Polar Maritime, Arctic, Polar, Continental, Tropical Martime.

- The names of the air masses indicated E, F, G and H at Appendix B are respectively:

  - Tropical  Maritime, Polar Continental, Tropical Continental Arctic.

  - Polar Continental, Tropical  Maritime,  Tropical Continental, Arctic,

  - Polar Continental,  Tropical Continental, Tropical Maritime  Arctic,.

  - Tropical  Maritime, Polar Maritime, Tropical  Continental, Polar  Maritime.

- Note- Following 6 Questions not feed in OMR

- In area L the main wet seasons will be:

  - At the equinoxes.

  - In January/February.

  - In July/August.

  - In November/December

- In area M in winter there will be:

  - Equatorial rains.

  - Extensive low cloud.

  - The Doldrums.

  - Dry trade wind conditions.

- In area N there will be:

  - Extensive winter rains.

  - Anticyclonic desert areas.

  - Dry summers and wet winters.

  - Polar front weather.

- In area 0 the climate will include:

  - The trade winds.

  - Dry warm summers and a wet winter season.

  - Steppe type with grassy plains.

  - A wet summer and dry cold winters.

- In area P the main weather factor will be:

  - Polar front depressions.

  - Depressions in winter, anticyclones in summer.

  - Extensive low cloud throughout the year.

  - Monsoon weather.

- In area Q  the climate will include:

  - Polar front depressions.

  - Cold anticyclonic weather.

  - Temperatures above zero for 3 months of the year.

  - Good visibility throughout the year.

- OBJECTIVE REVISIONS

- BELOW 2 QUESTIONS ARE NOT FEED IN OMR

- Figure shows the situation after an aircraft, equipped with INS, has passed over way point 2 and is tracking along the line TK( dashed). Using the information given in the diagram, and the fact that with DA/HDG selected on the control and display unit (CDU) of the INS, the display shows 6L/080, answer the following two questions:

- When DSRTK/STS is selected on the CDU, the left window will show:

  - 074.

  - 086.

  - 068.

  - 080.

- When XTK/TKE is selected on the CDU, the display will show ( to the nearest whole number):

- LEFT DISPLAY		RIGHT DISPLAY

  - 5L				080

  - 5R				6R

  - 5L				6L

  - 6R				5L

- Dynamic pressure is:

  - Static pressure minus pitot pressure.

  - Pitot pressure plus static pressure.

  - Density and static pressure.

  - Pitot pressure minus static pressure.

- When accelerating on an easterly heading in the N.Hemisphere, the magnet system of a direct reading magnetic compass will:

  - Turn clockwise, indicating an apparent turn towards the north.

  - Turn anti-clockwise, indicating an apparent turn towards the north.

  - Turn clockwise, indicating an apparent turn towards the south.

  - Turn anti-clockwise, indicating an apparent turn towards the south.

- The product of the second integration of the E/W accelerometer output in an INS system is

  - Longitude.

  - Departure.

  - E/W acceleration.

  - E/W ground speed.

- Density varies:

  - Directly with pressure and inversely with temperature:

  - Directly with temperature and pressure.

  - Inversely with temperature and pressure.

  - Inversely with pressure and directly with temperature.

- An aircraft is flying at TAS of 1100 kt and FL 650. A change of 0.1 mach causes a change in TAS of 57 kt. The temperature deviation at FL 650 assuming an ISA atmosphere is:

  - -6

  - +6

  - -3

  - +3

- “The deviating effect of vertical soft iron (VSI) ………. With decrease of magnetic latitude, due to the ……….. of H and the ………. Of Z”

- The line containing the words to correctly complete the above statement is:

  - Increase	decrease	increase.

  - Decrease	increase	decrease.

  - Decreases	increase	increase.

  - Increases	decrease	decrease.

- Angle of attack may be sensed by means of …………… mounted …………..

  - A stick pusher, adjacent to the flying controls.

  - An indexer, in the flight director.

  - A conical slotted probe, on the wing leading edge.

  - A hinged vane sensor, on the wing leading edge.

- The speed at the upper end of the Green are on the ASI is:

  - VRA

  - VNO

  - VNE

  - VMC

- “The rate gyro indicates the correct rate of turn when the ………….. precession due to the spring is --------------  to the ---------- rate of turn”.

- The line containing the words to correctly complete the above statement is:

  - Secondary		equal				aircraft.

  - Torqued		opposite			primary.

  - Primary		balanced by and equal	aircraft.

  - Secondary		opposite			aircraft.

- The servo altimeter is superior to the sensitive altimeter because:

  - It reduces barometric error.

  - It reduces high altitude error.

  - It reduces temperature error.

  - All of the above are correct.

- An aircraft is flying at FL 290, TAS 500 kt, mach number 0.86. The temperature deviation from ISA is:

  - -7

  - +7

  - -15

  - +25

- The advantages of the electrical artificial horizon over the air-driven artificial horizon are:

  - Random wander eliminated – greater rigidity.

  - Acceleration errors reduced – less rigidity.

  - Greater rigidity acceleration errors reduced – turning errors reduced.

  - Fast erection button fitted – quicker precession allowing faster indication – cut out switches reduce turning errors.

- The rotor of a rate gyroscope is over speeding. The pilot carries out a turn with the rate gyroscope indicating Rate 1. The actual rate of turn will be:

  - 3o per second.

  - More than 3o per second.

  - Less  than 3o per second.

  - 6o per second.

- OBJECTIVE REVISION NO. 5

- The latitude nut of a directional indicator (DI) is on the ………. Gimbal and causes the gyroscope to precess around its ……….. axis. The correct words to complete the above sentence are :

  - Inner		vertical.

  - Outer		vertical.

  - Inner		horizontal.

  - Outer		horizontal.

- If the rpm of the rotor in a turn and slip indicator is higher than normal, the turn indicator will:

  - Over read the correct rate of turn.

  - Under read the correct rate of turn.

  - Not indicate due to the increased rigidity.

  - Indicate correctly.

- A space gyro has …………. Gimbal ( s) and is ……….. so as to maintain alignment with ………

  - One	gravity 		a horizontal earth reference

  - Tow	case leveled	the horizontal

  - One	uncontrolled	a fixed space reference

  - Two	uncontrolled	a fixed space reference

- The rigidity ( gyroscopic inertia) of a gyroscope may be increased by:

  - Increasing the number of gimbals and decreasing the number of planes of rotation.

  - Increasing the speed of rotation and decreasing the mass of the rotor.

  - Increasing the speed of rotation and increasing the mass of the rotor.

  - Decreasing the speed of rotation and increasing the mass of the rotor.

- An electrically driven artificial horizon considerably reduces the acceleration errors of the air driven variety by:

  - Being less pendulous and more rigid.

  - Being more rigid and incorporating erection system cut-out switches.

  - Using mercury switches and torque motors.

  - Increased gyro rpm and giving it the opposite rotation direction.

- A blockage in the static line to the VSI will subsequently cause the instrument to display:

  - A decreased rate of climb.

  - An increased rate of climb.

  - A zero rate of climb.

  - An increased rate of descent.

- Machmeter readings are subject to:

  - Temperature error and pressure error.

  - Density  error and instrument error.

  - Temperature error and density error.

  - Instrument  error and position  error.

- In an INS which is Schuler turned, the largest unbounded errors are:

  - Due to the output of the first stage integrators.

  - Due to the real wander of the platform gyroscopes.

  - Due to accelerometer errors.

  - Track errors due to initial misalignment.

- Temperature error in the altimeter is due to:

  - The difference between the actual mean temperature below the aircraft and the mean temperature that would be found in ISA.

  - The difference between the actual pressure setting and 1013.2 mb.

  - Inaccurate calibration of the temperature compensating device.

  - The ambient temperature at which the aircraft is flying.

- An aircraft fitted with a direct-reading compass is turning from 315o through north on to 045o in the southern hemisphere. The direction of turn of the magnet system and the effect of liquid swirl on the error due to the turn are:

- TURN		LIQUID SWIRL

  - Anti-clockwise	reduce.

  - Clockwise 		reduce.

  - Anti-clockwise	increase.

  - Clockwise 		increase.

- The inertial platform in a north-referenced inertial navigation system is torqued to perform like a Schuler pendulum, so that when the platform moves over the earth:

  - Gyro drift errors are cancelled out.

  - The platform will always oscillate with respect to true north.

  - The platform remains level and aligned regardless of any aircraft accelerations.

  - Answers b and c are correct.

- An INS-equipped aircraft flies from 56o N 20 o W ( way point 3) to 56 o N 30 oW ( way point 4). Use this information to answer the next two questions.

- The initial track at way point 3 is:

  - 086 o T

  - 082 o T

  - 274 o T

  - 278 o T

- With constant drift during flight the aircraft’s heading will:

  - Increase by more than 10o.

  - Decrease  by less than 10o.

  - Increase by less than 10o.

  - Remain constant.

- Refer to Figure of a machmeter:

- The correct descriptions of the components labeled A, B and C in order, are:

  - Pitot pressure entry; airspeed capsule; ratio arm.

  - Static pressure entry; altimeter capsule; ranging arm.

  - Dynamic pressure entry; airspeed capsule; ratio arm.

  - Pitot pressure entry; altimeter capsule; ranging arm.

- In an inertial-lead VSI the source of the most pronounced error is:

  - Instrument.

  - Position.

  - Steep turn.

  - Missed approach manoeuvre.

- If an aircraft descends from FL 310 at a constant CAS, the Mach No. will:

  - Increase because the density error is increasing.

  - Decrease because static pressure is decreasing.

  - Decrease because the density  is increasing.

  - Decrease because static pressure is increasing.

- In an inertial reference system the gyros and accelerometersf are:

- GYROS			ACCELEROMETERS

  - Non-strap down		non-strap down.

  - Strap down		strap down.

  - Strap down		non-strap down.

  - Non-strap down		strap down.

- Equivalent Air Speed (EAS) is:

  - IAS corrected for compressibility.

  - CAS corrected for compressibility.

  - TAS corrected for compressibility.

  - TAS obtained from indicated Mach No. corrected for pressure and instrument error.

- In a turn and slip indicator the largest errors will be induced by:

  - Yaw.

  - Angle of bank.

  - Primary torque.

  - Reduced spring tension.

- If the pitot line becomes blocked during a climb the machmeter will:

  - Be unaffected.

  - Under read.

  - Over read.

  - Under read by an increasingly erroneous amount.

- In a direct reading compass the maximum errors will occur when turning through:

  - N/S high latitudes.

  - N/S low latitudes.

  - E/W low latitudes.

  - E/W high latitudes.

- With reference to the flux valve of a gyromagnetic compass:

  - The flux valve is pendulously mounted and is free to turn so that it remains aligned with the earth magnetic field.

  - The flux valve is fixed to the aircraft and so turns with the aircraft to measure the angle between the aircraft and the earth magnetic field.

  - The flux valve is not subject to acceleration errors.

  - The flux valve is pendulously mounted and so is not affected by the vertical component of the earths magnetic field.

- The purpose of the annunciator circuit in a remote indicating compass is:

  - To align the gyro unit with the aircraft heading suing the output from the rotor of the signal selsyn.

  - To monitor the signals sent to the precession coil to ensure the gyro is turned the correct way to bring the rotor coil in line with the earths magnetic field.

  - To indicate the correct way to turn the synchronizing knob to initially synchronise the compass system and then to indicate correct operation of the system.

  - To monitor the signals sent to the precession coil to ensure the gyro is turned the correct way to make its axis line up with the electrical field in the signal selsyn.

- An increase of 0.15 Mach result in an increase of 93 kt TAS of an aircraft. Using this information answer the following two questions.

- The local speed sound is:

  - 560 kt.

  - 685 kt.

  - 620 kt.

  - 580 kt.

- If the temperature deviation from ISA is + 5oC, the approximate flight level is:

  - FL 200.

  - FL 150.

  - FL 220.

  - FL 250.

- Total air temperature is:

  - The temperature of air which has suffered the full effect of compression heading.

  - The ambient air temperature.

  - Static air temperature minus ram air temperature.

  - True outside air temperature allowing for cooling.

- An aircraft is carrying out a rate one turn at a TAS of 480 kt. The diameter of the turn will be:

  - 2.5 nm.

  - 15 nm.

  - 5 nm.

  - 10 nm.

- OBJECTIVE REVISION NO. 6

- Total Air Temperature is:

  - The maximum temperature attainable by the air when brought to rest, adiabatically.

  - The temperature indicated on the air temperature thermometer plus the ram rise.

  - The static air temperature minus the recovery factor.

  - The recovery factor plus the ram rise.

- The difference between static air temperature and total air temperature is known as:

  - Corrected outside air temperature.

  - The ram rise.

  - The recovery factor.

  - Hot ramp radiation.

- An Air Data Computer has inputs of:

  - Dynamic pressure, static pressure and air temperature.

  - Pitot pressure, static pressure and static air temperature.

  - Static pressure, pitot pressure and total air temperature.

  - Dynamic pressure, static pressure and ram air temperature.

- The following symbols, A, C and E are best described respectively as:

- A			    B		C	       D	          E

  - Off route way point, airport, navigation aid.

  - Next way point, navigation air, airport.

  - Off route way point, navigation aid, a navigation point making up selected route.

  - Active way point aircraft currently navigating to, navigation aid, off-route way point.

- When using EHSI, weather radar may be displayed on following settings:

  - MAP, centre map, plan, nav, VOR/ILS.

  - VOR/ILS, map, nav, plan.

  - Nav, map, VOR/ILS, plan.

  - Expanded nav, map, expanded VOR/ILS.L